Inspection Check List for Amateur Built Aircraft

The following checklist was generated for use during my inspection of Amateur Built aircraft.  Feel free to print it out and share it with your friends.  This is NOT an official source of information.  Always contact your local FAA office for official information or reference FAA Order 8130.3 current revision.

This document has been developed as a basic inspection guideline for certification of amateur-built aircraft.  It is not intended to be all-inclusive for every variation of experimental amateur-built aircraft.  But, it is general enough in scope to show that the certification inspection is not to be a detailed Annual/100 hour type inspection since the builder/applicant has already declared that the aircraft is "airworthy" by signing the application for the airworthiness certificate and making a signed log book statement.  The FAA/DAR issues the special airworthiness certificate for operating the amateur-built aircraft.  The use of the following guide, you should be able to perform what amounts to an in-depth pre-flight that should reasonably assure that the aircraft will operate as intended.

Checklist:

Builder: _____________________________ Model _________________________ S/N ________________

Date of Inspection_____________________ Inspector ______________________

Regulatory Prerequisites & Placards

Satisfactory /Unsatisfactory Item 							S	U

- Meets 51% rule, has invoices, plans, pictures and building records				Yes  No

- Presents notarized statement FAA Form Eligibility Statement 				Yes  No

- Presents FAA Form 8130-6 Application for A/W Certificate properly filled out 		Yes  No

- ID data plate is fireproof (steel), has Builder's name, model, serial number. 			Yes  No

- ID data plate is installed in proper external location per FAR 4§5. 11(a). 			Yes  No

- N-Numbers installed, block letters, 3" proper location per FAR §45.25, or 12 inch, if a/c has cruise speed over 180 Knots CAS. 									Yes  No

- Word Experimental Displayed, 2" min size, proper location per FAR §45.23(b). 		Yes  No

- Passenger warning placard (not required for single place) ".  This aircraft does not meet U.S. safety standards for certified aircraft..." 								Yes  No

- Registration certificate available in aircraft. (NOT pink slip!) 				Yes  No

- Builder makes statement in logbook that aircraft has been inspected 			Yes  No

- A/C Weight and balance done, and shows within designer's limits.  FAR §91.9 Placards 	Yes  No

- Empty weight

- Gross weight

- Most forward CG location

- The completed weight and balance report, including load limits for flightcrew, oil, fuel, baggage, should be available in the aircraft along with other applicable placards, listings, and markings FAR §91.0.

- ALL controls - flight and engine, marked as to use. 					Yes  No

- Markings for:

- Throttle- open-closed 								Yes  No

- Carb Heat- pull on 								Yes  No

- Flaps- Up- Down, Degrees/Inches Take Off Setting Marked 				Yes  No

- Trim Tab- Nose Up, Nose Down, Take Off setting					Yes  No

- Trim Tab- rudder left, rudder right, neutral						Yes  No

- Mixture- Push rich								Yes  No

- Fuel On-Off levers								Yes  No

- Fuel quantity and type marked on or near each cap 					Yes  No

Cockpit interior

- Seat Belts function, and angle is slightly rearward 					Yes  No

- Shoulder harness function, and angle is -5 to + 30 degrees 				Yes  No

- Seat Belt Anchor Points firm, no interference 						Yes  No

- Shoulder Harness Anchor Points firm, no interference 					Yes  No

- Seats and seat tracks o.k., stops on aft of rails 						Yes  No

Instruments and required equipment

- ELT meeting TSO-C91A properly installed, with remote switch and battery date current and recorded in aircraft records (See Checklist explanations Page) for details FAR §91.205 and others 		Yes  No

- Fuel gauge each tank, has been calibrated and calibration in records showing unusable fuel 	Yes  No

- VFR Day requirements: * Note: All instruments should be marked with green/red line ranges 	Yes  No

- Altimeter 									Yes  No

- Airspeed indicator 								Yes  No

- Tachometer 									Yes  No

- Oil Pressure 									Yes  No

- Oil Temperature									Yes  No

- Compass 									Yes  No

- For retract gear aircraft, indicator of up/down gear 					Yes  No

- VFR Night Requirements- Day VFR plus:

- Position Lights									Yes  No

- Anti collision strobes/rotating beacon which meets FAA standards			Yes  No

- Spare fuses									Yes  No

- Electrical energy sufficient for duration of aircraft range, plus reserve 			Yes  No

- IFR Requirements Day & Night VFR, plus:

- Working two-way radio (See Checklist explanations Page)				Yes  No

- Gyroscopic rate of turn indicator (Turn & Bank or Turn Coordinator)			Yes  No

- Slip / Skid indicator 								Yes  No

- Sensitive altimeter								Yes  No

- Artificial horizon									Yes  No

- Directional gyro 									Yes  No

- Clock										Yes  No

- Vacuum gauge, if vacuum powered, or volt and amp meter, if electric (See Checklist explanations Page)											Yes  No

- Heated Pitot									Yes  No

- Generator or alternator								Yes  No

- Alternate Static Source (See Checklist explanations Page) Systems			Yes  No

- Wire type and size is appropriate for load being carried, and connections are solid.  All installed systems perform as intended.								Yes  No

Electrical

- Battery & Electrical System, wiring adequate size and secure				Yes  No

- Switches marked for operation, and wired properly 					Yes  No

- Circuit Breakers or fuses labeled for value and function, and wired properly			Yes  No

- Ground on battery to airframe, or wired into place properly 				Yes  No

- Ammeter										Yes  No

- Voltmeter 										Yes  No

- Landing Lights / Position Lights wired correctly						Yes  No

- Cockpit lights/ Instrument lighting 							Yes  No

- Battery case and battery securely mounted						Yes  No

- Battery vented overboard								Yes  No

# Avionics

- Antennas are properly installed, and have proper support/doubler plates			Yes  No

- Coax Cable secured, with slack enough to prevent disconnection 				Yes  No

- Radios/Avionics and cooling fans are mounted securely					Yes  No

- Avionics gear is wired properly, and functions						Yes  No

- Transponder "Mode C" check done, and in aircraft records 				Yes  No

Fuel System

- Caps fit, and are vented on all tanks							Yes  No

- Fuel drains installed at lowest portion of tank, and at lowest point in fuel lines		Yes  No

- Fuel Vent System contains no loops which would block venting				Yes  No

- Shutoff valve/ Selector switch ease of operation, and clearly marked for intended operation	Yes  No

- Fuel Line routing proper material and diameter for intended fuel supply			Yes  No

- Fuel Strainer functional and safety wired

- Fuel Lines protected from chafing, and secure from "catching a foot."(Checklist explanations Page)													Yes  No

- Fuel line routing avoids areas of heat							Yes  No

Gear/Wheel/Brake Systems

- Retractable Gear functions- Operations check						Yes  No

- Emergency gear extension test								Yes  No

- Clearance in wheel wells								Yes  No

- Brake System line routing, reservoir, pumps 						Yes  No

- Brake & Wheel installation secure							Yes  No

- Taxi test on gear toe in/ toe out o.k.							Yes  No

- Tires are clear of pants or struts, including clearance of "mud scraper", no binding		Yes  No

- Wheel Pants are secure								Yes  No

- Tail Spring secure, well designed, and is clear of rudder					Yes  No

Control Surfaces

A. Designers Recommended limits: This Plane Measures:

Aileron Up travel: ______		 Degrees/Inches _______ 		Degrees/Inches

Down travel : __________		Degrees/Inches _______ 		Degrees/Inches

Elevator Up travel: _______	Degrees/Inches _______ 		Degrees/Inches

Down travel: ___________	Degrees/Inches _______ 		Degrees/Inches

Rudder Left/Right ______ 	Degrees/Inches _______		 Left _______ Right __________

General Condition- Fuselage, Wing, Tail Assembly

- Skin condition, wrinkles, rivets, or tape							Yes  No

- Vertical Fin, movement and condition							Yes  No

- Elevator assembly, movement and condition						Yes  No

- Trim Tabs function, no binding, heavy structure to reduce flutter potential			Yes  No

- Control Travel "Stops"								Yes  No

- Control Cables Safety Wired Correctly							Yes  No

- Counterweights secure								Yes  No

- Doors close securely and open properly, if open in flight, markings				Yes  No

- Windows secure, and markings for distance if open in flight 				Yes  No

- Canopy secure									Yes  No

- Canopy emergency release marked red and copper safety wired 				Yes  No

- Canopy locks/door locks work								Yes  No

Flight Controls

- Proper tension of cables								Yes  No

- Safety wiring of cables								Yes  No

- Clearance on controls/no hitting panel or floorboards					Yes  No

- Freedom of movement								Yes  No

## PROPER DIRECTION

Is everything safetied?  Engine Compartment 						Yes  No

- Engine Compression in Logs 1=___ 2=____ 3=____ 4=____ 5=____ 6=___ 7=____8=____9=____

- Throttle cables anchored and functional, spring open					Yes  No

- Clearances in engine compartment							Yes  No

- Carb Heat Control- Functional								Yes  No

- Mixture Control- Functional, spring full-rich						Yes  No

- Fuel Hose routing to avoid heat & bends (Fire sleeve preferred)				Yes  No

- Fuel strainer safety wired								Yes  No

- Fuel Hose diameter sufficient for engine							Yes  No

- Fuel and Oil Hoses proper material							Yes  No

- Oil System Hose & Cooler installation							Yes  No

- Prop Governor. Functional, proper lines							Yes  No

- Firewall steel, no openings								Yes  No

- Engine Mount secure, no cracks							Yes  No

- Alternator, belts, accessories properly installed						Yes  No

- Heater hoses proper material								Yes  No

- Heater design minimizes CO poisoning chances						Yes  No

- Baffles appear correct									Yes  No

- All electrical and ignition wires appear correct						Yes  No

- Cowling secure, (INSTALL IT NOW).							Yes  No

Propeller

- Propeller nicks									Yes  No

- Safety Wired Bolts of sufficient size							Yes  No

- Bolts torqued properly								Yes  No

- Bolts clear engine, proper length							Yes  No

- Prop clears ground in takeoff attitude by >7"						Yes  No

- Prop Spinner o.k., clear of cowl, not uneven spin						Yes  No

- Ready for Run-up.									Yes  No

Test Run Engine

Note: At least one hour of runup should have been done previously, and logged in the aircraft logbook.  The engine compressions should also be recorded after one hour of runup.  Some may be low on a fresh overhaul engine, but it is important to get this information as a base line!

- Engine starts easily									Yes  No

- Oil Pressure comes up quickly to proper level						Yes  No

- Oil Temp comes off cold peg								Yes  No

- Check all instrument operations/tach/oil/volts/ etc.					Yes  No

- Record all data five minutes after startup here

__________________________________________________________________________________________________________________________________________________________________________________________

- Carb Heat Functions, RPM Drop is ________						Yes  No

- Mag Drop, Left is ______ Right is ________ 						Yes  No

- Fuel Gauges show calibrated 								Yes  No

- Mixture/Throttle/Prop controls work							Yes  No

- Fuel Shutoff works									Yes  No

- No abnormal vibration								Yes  No

- Propeller tracks within specs								Yes  No

- Airworthiness Directive Compliance							Yes  No

- Engine AD's which apply have been complied with, and how (Attach list if necessary)	Yes  No

- Equipment AD's which apply have been complied with, and how (Attach list if necessary) 	Yes  No

Certification

Required FAA Forms:

- 8130-6 Airworthiness Certification Application.

- 8130-12 Eligibility Statement  (Notarized Statement).

- Three view sketch, drawing, or photograph of proposed aircraft project.

- 8050-88 Affidavit of Ownership.

- 8050-1 Aircraft Registration application (should have hard card registration for OKC).

- Weight and Balance report (empty, gross, and most forward CG limit) §91.9.

- Flight test program §91.319(b) address goals and objectives of each test.

Aircraft records should have the following statement:

I certify that I have built this aircraft for my own education and recreation, and I have inspected it fully.  I am the manufacturer, and I consider it eligible for issuance of an Experimental Airworthiness Certificate for the purpose of operating amateur-built aircraft under provisions of FAR 21.191(g).

Builders name, signature, and date

- Discuss operating limitations with applicant, flight test areas, and flight test time.		Yes  No

- Airworthiness application properly filled out and signed?					Yes  No

- Sign Logbook and issue certificate, if applicable OR 					Yes  No

- Provide list of deficiencies to applicant, if applicable Unsatisfactory Conditions 		Yes  No

N____________		Owner ________________________________________________________________

The following unsatisfactory conditions were noted in your inspection.  It is your responsibility to correct these conditions prior to obtaining an airworthiness certificate.  Please correct the deficiencies and return this list, with evidence of the corrections, to the certificating inspector.

- Condition:________________________________________________________________________________

Corrective action taken: _________________________________________________________________________

_____________________________________________________________________________________________

- Condition: ________________________________________________________________________________

Corrective action taken:

_____________________________________________________________________________________________

Signed: __________________________________________  Date: _______________________________________

NOTE:  If all of the above items are satisfactory the FAA or DAR will issue a Special Airworthiness Certificate and endorse your airframe record book with the following statement:

I find that the aircraft meets the requirements for the certification requested in accordance with CFR §21.191(g) and have issued a Special Airworthiness certificated dated 00/00/2010.  The next inspection is 00/00/2011.

I.B. Good Signature

ASI or DAR