# Index

# Index....................................................................	1

Introduction…...……………………………………..	2

General Operating and Flight Rules FAR 91………...	2

Definitions……………………………………………	3

Primary Responsibility for Airworthiness FAR 91…..	4

Maintenance Preventive Maintenance

Rebuilding and Alteration FAR 43…..........……	4

Certification: Airmen other than

Flightcrew Members FAR 65…..........…............	4

Privileges of Certificates FAR 43, 65, 145….........….	5

Maintenance Required Far 91………………………..	6

Aircraft Inspections FAR 91, 43………………...…...	6

Other Aircraft Maintenance FAR 43…………………	7

Maintenance Records FAR 91, 43, 145……………...	8

Approved for Return to Service vs

Return to service FAR 43, 91……………..…….	10

Sample Log book Maintenance Record Entries……...	11

Advisory Circulars…………………………………...	12

FAA Web Server Access Information..........................	12

Malfunction or Defect Reports……………………….	13

Airworthiness Directive Record Form:

Sample Page………………………………..…....	14

For Owners Use (Detach & Staple to Records)………	15

General Aviation

Aircraft Maintenance Responsibilities

## Introduction

For optimum safety and freedom in aviation every airman should know the regulations pertaining to his individual responsibilities. In addition, every airman should be familiar with certain overlapping and relative rules that require cooperative efforts of operations and maintenance to properly discharge the shared responsibility of maintaining aircraft airworthiness.

This is a collective review of Federal Aviation Regulations relating to the individual and cooperative responsibilities of aircraft owners, operators, pilots, mechanics, inspectors, and repair stations for maintaining the airworthiness of general aviation aircraft.

We hope that this effort to “rightly divide” The FARs by collecting applicable sections from FARs 43, 65, 91, and 145 and “putting them all together” will accomplish a three fold objective.

- Establish a more knowledgeable rapport between operations and maintenance airmen and agencies.

- Prevent violations of the Federal Aviation Regulations pertaining to aircraft maintenance.

- Prevent aircraft accidents in which lack of maintenance or improper maintenance may be a factor.

(The brief comments that follow each FAR reference do not con-stitute a legal interpretation of the regulation, they are simply a paraphrased rendition of the primary content of that section.)

FAR 91 General Operating and Flight Rules

91.1           “Applicability” -Governs operation of aircraft.

91.7 (a) 	Aircraft must be airworthy to be operated.

91.7 (b) 	Flight to be discontinued if unairworthy Condition occurs.

Definitions

FAR 1	“Person” – An individual or company, corporation, etc.

FAR 1	“Operate” – Use for the purpose of air navigation.

FAR 1	“Maintenance” – includes, inspection, repair etc.,

BUT NOT PREVENTIVE MAINTENANCE

FAR 1	“Preventive Maintenance” – Simple, or minor preservation operations or replacement of small standard parts not involving complex assembly operations.

Airworthiness. US Code Title 49 spt. 44704 and FAR Sections

21.183(a), (b), and (c) all relate to the two conditions necessary for

issuance of an airworthiness certificate. The statutory language

establishes the two conditions as: (1)  The aircraft must conform

to the type design (certificate); and (2) it is in a condition for safe

operation. The above conditions are further reflected as terms and

conditions appearing upon the front of the Standard Airworthiness

Certificate, FAA Form 8100-2.

Discussion. a. The aircraft must conform to its type design (cer-

tificate). Conformity to type design is considered attained when the

required and proper components are installed and they are

consistent with the drawings, specifications, and other data that is

a part of the type certificate. Conformity would include applicable

supplemental type certificates and field approved alterations.

b. The aircraft must be in condition for safe operation. This refers

to the condition of the aircraft with relation to wear and

deterioration. Such conditions could be skin corrosion, window

delamination/crazing, fluid leaks, tire wear, etc.

Conclusion. An aircraft can be considered to be airworthy when

the Administrator finds it conforms to the specifications of its type

certificate, and it is in a condition for safe operation. If one or both

of these conditions are not met, the aircraft would be unairworthy.

FAR 91 – Subpart C – Primary Responsibility for Airworthiness

91.401	Aircraft must be maintained “within or without” the U.S.

91.403 (a)	Owner or operator is primarily responsible for maintaining

airworthiness, including A.D. compliance.

91.403 (b)	Prescribed maintenance and persons authorized to perform it.

39.3           Operate in accordance with A.D.

FAR 43 – 	Maintenance, Preventive Maintenance, Rebuilding and Alteration

43.1           Applicable to all certificated aircraft except expermental aircraft

that have never been issued any other kind of certificate.

43.3           Persons authorized and work they are authorized to perform.

*Preventive maintenance items are listed in app. “A”

43.3 (b)     Certificated mechanics may perform as prescribed in FAR 65.

43.3 (c)      Certificated repairman may perform as prescribed in FAR 65.

FAR 65 Certification: Airman Other Than Flight Crew Members

65.81 (a)	Mechanic – General privileges and limitations.

* Mechanics may not perform major repair or major alteration of propellers and any repair or alteration to instruments.

* Must have satisfactorily performed or shown his ability to perform before supervising or approving for return to service.

65.81 (b)	Mechanics must understand current instructions on how to do a specific job before it is attempted.

65.83	At least 6 months in the preceding 24 months or otherwise qualified by the FAA. Recent experience required.

65.85	Airframe rating: Additional privileges, 100 hour inspection.

65.87	Powerplant rating: Additional privileges, 100 hour inspection.

  - Certificate is to be kept where the mechanic normally works.

PRIVILEGES OF CERTIFICATE

65.95 (a)	Inspection authorization: Privileges and Limitations

* Inspect and approve for return to service major maintenance if

done in accordance with approved data.

* Perform annual inspections.

*Perform or supervise progressive inspections.

65.95 (b)	Authorization to be kept available for inspection by aircraft owner or mechanic.

65.103 (a)	Repairman Certificate: Privileges and limitations - may perform or supervise the specific job for which he is employed and certificated in a Repair Station.

65.104	Repairman Certificate: Experimental Aircraft Builder may perform Maintenance and Condition inspection on an aircraft he has built.

43.3 (d)	Persons working under the supervision of certificated mechanic or repairman.

*Supervisor personally observes to the extent necessary to ensure

that the work is done properly.

*Supervisor is readily available in person for consultation.

*100 hour and annual inspections may not be supervised.

43.3 (e)	Repair stations may perform as provided in FAR 145.

43.3 (f) & (g) Large air carrier and commercial operators.

43.3 (g)	Pilot may perform preventive maintenance on aircraft owned or operated by him, not used in air taxi service.

145.51	Privileges of certificate. (Repair Station)

145.51 (a)	Maintain or alter items for which it is rated.

145.51 (b)	Approve for return to service after maintaining and altering.

145.51 (c)	Airframe rated Stations may perform 100 hours, annual and progressive inspections and return aircraft to service.

145.51 (d)	Maintain or alter articles for which rated at other places under certain conditions.

* May not approve for return to service any major repair or alteration unless work is done in accordance with technical data approved by the FAA.

145.53	May not maintain or alter an article if it requires special technical data, equipment, or facilities that are not available to it.

145.55	Must provide personnel, facilities, equipment equal to current standards for issuance of certificate.

145.57 (a)	Perform its operations in accordance to the standards of FAR 43.

* Preventive maintenance items are listed in FAR 43

Appendix "A", Para. (c).

FAR 91  Maintenance Required

91.405	This is the general aviation "maintenance program".

-Three distinct owner operator responsibilities to ensure continual aircraft airworthiness.

1. Owner or operator shall have aircraft inspected.

2. Owner or operator shall have defects repaired between inspections.

3. Owner or operator shall insure that maintenance personnel make appropriate entries in Maintenance Records.

AIRCRAFT INSPECTIONS

91.409 (a)(1) Annual inspection within the preceding 12 calendar

months.

* 100 hour inspection may not be substituted for an annual unless performed by authorized person and recorded as an annual.

91.409 (b)	100 hour inspection required to carry persons for hire or give flight instruction for hire.

*10 excess time allowed, if necessary. Any excess time must be included in next 100 hour cycle.

WARNING This allowance does not apply to ADs.

91.409 (c)	Annual or 100 hour inspection not applicable if owner or operator complies with progressive or continuous inspection under paragraphs (d) and (e) or aircraft inspected under FAR123, 125 or 135.

43.15 (a)	Performance rules for inspections.

*To determine whether aircraft meets all applicable airworthiness requirements, (including A.D. compliance).

43.15 (b)	Rotorcraft inspected in accordance with manufactures maintenance manual.

43.15 (c)(1) For 100 hours and annual inspections an inspection    checklist that     covers the scope and detail of FAR 43,  Appendix "D" shall be used.

43.15 (c)(2) Person approving for return to service shall determine satisfactory "run up" performance.

43.16	Each person performing an inspection or other maintenance specified in an airworthiness limitations section of the manufacturers maintenance manual must do it in accordance with that manual.

91.403 (c) 	"Airworthiness" must be complied with if the aircraft is to be operated. (Parts replacement times, inspection intervals etc.)

Other Aircraft Maintenance

43.13 (a)	Maintenance performance rules (general)

*Use methods, techniques and practices acceptable to the FAA.

*Use tools, equipment and test apparatus to assure that work is done

according to acceptable industry practices.

Use special test equipment recommended by manufacturer or the equivalent acceptable to the FAA.

43.13 (b)	The work done and materials used must be of such quality that the condition of the aircraft is equal to its original or properly altered condition.

Maintenance Records

91.417	Registered owner or operator shall keep the records in item #1 below

until the work is repeated, superceded, or for one year.

(1.) Records of maintenance, alterations, required or approved inspections, as appropriate, for each aircraft, engine, propeller, rotor,

and appliance must include:

(i)    A description of the work.

(ii)   The date of the work.

(iii)  Signature and certificate number of person approving work for

return to service (Including the pilot owner/operator for preventive maintenance).

(The foregoing record entries must be made by the person performing the work- 43.9)

(2) The following records must be retained and transferred with the

aircraft when sold.

(i)    Airframe total line in service.

(ii)   Current status of life limited parts.

(iii)  Time since overhaul on items required to be overhauled.

(iv)   Current status of Airworthiness Directives including method of compliance and if repetitive the next due date and/or time.

(vi)   List of current major alterations.

91.417 (c)	The owner or operator must make the maintenance records available for inspection by the FAA or NTSB.

91.419 	"Transfer of Maintenance Records".

(The records in item #2 above must be transferred to the new owner and those specified in item #1 must be transferred also unless arrangements are made with the seller to make them available to theFAA or NTSB on request.)

43.9 (a)	The person who does the work shall make an entry in the maintenance records containing:

(1) A description of the work performed (or reference to a 337 or work order is applicable).

(2) Date of completion of the work.

(3) The name of person performing the work.

(4) If approved for return to service the signature and certificate number of person who approved it.

* In addition, major repair and major alterations are to be entered on a separate form.

145.59 (a)	A qualified inspector inspects before approval for return to service; the repair station certifies airworthiness and approves for return to service.

145.61	Repair Station maintains adequate records of work it does, naming the person that does the work and the inspector of the work.

43.11	Content, and form of record entry for inspections conducted under 91,123,125,135.411 (a)(1), 135.419.

*The person approving or disapproving for return to service makes entry in the permanent maintenance record including the following information.

(1) Type of inspection (annual, progressive, manufacturers recommended, owners approved, etc.).

(2) The date and aircraft time in service.

(3) The signature, kind of certificate number held by the person approving or disapproving the work.

(4) Statement certifying airworthiness if approved (See Figure 3, Page 13)

(5) Statement certifying unairworthiness if not approved (if not approved owner must be given a list of discrepancies).

APPROVED FOR RETURN TO SERVICE VS RETURN TO

SERVICE

43.7 (a)	Persons authorized to approve for return for service as provided

in FAR 65, 145, etc.

43.5 (a)	No person may return to service an aircraft or

article that has undergone maintenance unless:

(1) It has been approved for return to service.

(2) Permanent maintenance record entries have been

made. (including preventive maintenance)

(3) The major repair or alteration form has been executed.

(4) Any change in operating limitations or flight data is revised and set forth as prescribed in FAR 91.9.

91.9 (a)	Operating limitations shall be complied with during operation.

91.9 (b)	Operating limitations (aircraft flight manual, placards,

listings, W&B markings, etc.) must be current and available

in the aircraft during operation, including:

(1) Powerplant markings and placards.

(2) Airspeed markings and placards.

(3) Aircraft weight and balance information and other...

* Approval for return to service is a maintenance record entry by an authorized individual.

** Return to service is any action by any person to put an aircraft or article into an operational status after it has been maintained or altered.

LOGBOOK MAINTENANCE RECORDS

The FAA is responsible for the regulation and promotion of civil aviation in such a manner as to best foster its development and safety.

FAA aircraft maintenance responsibilities are accomplished by the general aviation maintenance inspector in his day to day activities of certification and surveillance of aircraft, maintenance airman and agencies. He is also charged with the investigation and reporting of aircraft accidents, incidents, malfunctions, defects, and violations of the Federal Aviation Regulations. Of our many interesting job functions, the least desirable ones are accident investigation and violation investigation. We hope that this "aircraft maintenance responsibilities" program will help prevent your involvement in either accident or violation.

Responsible aircraft maintenance requires a cooperative effort on the part of all of us who have aircraft maintenance responsibilities.

SAMPLE LOGBOOK/ MAINTENANCE RECORD ENTRIES

February 9, 1966. Total time 435 Hours. Complied with AD 64-27-2 by installing new

rubber float, and new bowl cover screws.

Inspected solder on float valve bracket and found okay. Stamped - 64 on nameplate.

George B. Jones IA 272182

Figure 1.

Typical entry for compliance with an airworthiness

Directive with a one- time compliance.

February 9, 1966. Total time 352 hours. complied with AD 61-23-01 paragraphs a(1) and

a(2) by tapping and magnifying glass. No cracks found. Void on top of blade #2, B2-

248-53A, S/N 123, is 2" long and extends from 25" to 27" outboard of blade butt rib.

Next inspection due at 377 hours.

George B. Jones IA272182

Figure 2

Typical entry for compliance with an Airworthiness

Directive with recurring inspection.

SAMPLE LOGBOOK/ MAINTENANCE RECORDED ENTRIES

January 18, 1967. Total aircraft time 853.00 hours. Tach reading 420.80 Replaced right main wheel bearing, P/N 19844, upper bushing in R&L landing gear frames, both brake hoses, P/N 34052, and bled brakes. I certify that this aircraft has been inspected in accordance with an annual inspection and was determined to be in airworthy condition.

George B. Jones IA272182

Figure 3

Sample logbook entry for a typical annual inspection when the aircraft

is found to be "airworthy". Note that the date, aircraft total time, and

tach or recorder reading are included. The tach or recorder reading should not be confused with the total time and should only be shown

IN ADDITION to the total time entry. The mechanic has indicated he holds an Inspection Authorization by prefixing his certificate number with the letters "IA".

January 18, 1967.Total time 853.00 hours Tach reading 420.80 I certify that this aircraft has been inspected in accordance with an annual inspection and a list of discrepancies and unairworthy items dated (insert date) has been provided for the aircraft owner or leasee.

George B. Jones IA272182

Figure 4

Required entry for Annual inspection when aircraft is found to be

"unairworthy".  that the date, total time and tach reading are included.

January 18, 1967. Total aircraft time 853.00 hours, Tach reading 420.80 changed oil and oil filter and cleaned spark plugs.

George B. Jones Comm.272182

Figure 5

Typical entry for pilot owner/operator accomplished preventive maintenance.

Advisory Circulars

Explanation of AC system. The FAA issues advisory circulars to inform the aviation public in a systematic way of nonregulatory material of interest. Unless incorporated into a regulation by reference, the contents of an advisory circular are not binding on the public. Advisory circulars are issued in a numbered subject system corresponding to the subject areas of the Federal Aviation Regulations (FAR's) (Title 14, Code of Federal Regulations, Chapter 1, Federal Aviation Administration). An AC is issued to provide guidance and information in its designated subject area or to show a method acceptable to the Administrator for complying with a related Federal Aviation Regulation.

The Advisory Circular Checklist AC 00-2.XX or subsequent issues contain instructions on how to obtain the desired advisory circulars.

FAA Web Server Access Information

The Federal Aviation Administration's main server is available at http://www.faa.gov. It hosts information from various FAA Organizations. The advisory circular checklist is available at:

http:www.faa.gov/agc/ac-chlst/actoc.htm.

MALFUNCTION OR DEFECT REPORT

Your help is needed to reduce the growing loss of valuable Malfunction or Defect information now occurring in the Service Difficulty Report and the General Aviation Airworthiness Alerts program. A number of the reports (approximately 15 percent) received each day are on aeronautical products that cannot be directly associated with a specific aircraft, powerplant, or propeller. As a result, there is no way to bring the problem to the attention of the manufacturer having prime interest in the reliable operation of that component, accessory, or appliance.

Aeronautical products in the component, accessory, and appliance categories are numerous in just about every certificated aircraft currently in operation. Further, a high percentage are not individually approved by the Federal Aviation Administration. Their acceptance is, in most cases, based on having been installed on the aircraft or power plant at he time of type certification or subsequent supplemental type certification. Under this arrangement, compliance with applicable regulations are the responsibility of the type certificate holder. For this reason, it is very important that the Malfunction and Defect report contains the identity of the aircraft or power plant on which the aeronautical product is installed.

Advisory Circular AC 20-109 contains an explanation of this program.

Advisory Circular AC 43-16A  Publishes General Aviation Airworthiness Alerts.

It is available at the AFS-600 Home Page address;

http://www.mmac.jccbi.gov/afs/afs600

AIRWORTHINESS DIRECTIVE COMPLIANCE RECORD

Make Piper                                Model PA28-161                             S/N 28-7410669______________

| AD Number | Subject | Date and Hours of Compliance | Method of Compliance | One time | Recurring | Next Comp. Due. | Authorized Signature |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 62-19-03 08-28-62 | Prop Bolt failure | 1044.8 12-01-77 | N/A by S/N | X |  |  | George B. Jones AP 272182 IA |
| 64-06-06 04-06-64 | Control Wheel failure | 1044.8 12-01-77 | N/A by S/N | X |  |  | George B. Jones AP 272182 IA |
| 67-20-04 09-27-67 | Main landing gear torque link failure | 1044.8 12-01-77 | N/A by Torque links not drilled for lube fittings | X |  |  | George B. Jones AP 272182 IA |
| 67-26-02 05-22-68 | Various Modifications | 1044.8 12-01-77 | N/A by S/N | X |  |  | George B. Jones AP 272182 IA |
| 77-23-03 11-14-77 | Control Rod Binding | 1044.8 12-01-77 | C/W by installing new style rod end | X |  |  | George B. Jones AP 272182 IA |
| 79-02-05 01-29-79 | Fuel Flow Interruption | 1308.7 12-30-79 | N/A by S/N | X |  |  | George B. Jones AP 272182 IA |
| 79-13-03 06-08-79 | Prevent Potential Fire Hazard | 1352.2 01-05-80 | N/A by S/N | X |  |  | George B. Jones AP 272182 IA |
| 79-22-02 10-26-79 | Prevent Possible Fuel Leakage and Fire Haz. | 1352.6 01-05-80 | N/A by S/N | X |  |  | George B. Jones AP 272182 IA |
| 80-14-03 07-01-80 | Disruption of Radio Communication | 1422.3 01-05-81 | N/A by S/N | X |  |  | George B. Jones AP 272182 IA |
| 81-23-05 03-08-82 | Prevent In-Flight Fire | 1615.4 05-25-82 | C/W by inspection and by installing Piper kit P/N 764-303V | X |  |  | George B. Jones AP 272182 IA |
| 95-26-13 02-05-96 | Oil Cooler Hose Failure | 2589.2 03-28-96 | C/W by installing new hoses |  | X | 3589.2 03-28-04 | George B. Jones AP 272182 IA |
| 96-10-03 06-14-96 | To Prevent Flap Handle Bolt Failure | 2596.9 07-15-96 | C/W By Inspection and by installing new bolt & bushing | X |  |  | George B. Jones AP 272182 IA |

AIRWORTHINESS DIRECTIVE COMPLIANCE RECORD

Make _________________________ Model __________________  S/N________________________________

| AD Number | Subject | Date and Hours of Compliance | Method of Compliance | One time | Recurring | Next Comp. Due. | Authorized Signature |
| --- | --- | --- | --- | --- | --- | --- | --- |