Light Sport Certification

87. APPLICATION FOR AIRWORTHINESS CERTIFICATE.

Form 8130-6 is required whenever an airworthiness certificate is issued or amended. This includes changes to operating limitations that may have been prescribed. The applicant must complete the appropriate sections and sign the application. A program letter also must be submitted to the FAA with any other document(s) required for the requested certification. The program letter must be reviewed to ensure all of the requirements of § 21.193(d) have been met.

88. CERTIFICATION PROCEDURES.

The following procedures are common for issuance of Form 8130-7, consistent with any other specific procedures that may be prescribed in other paragraphs dealing with individual airworthiness categories. In no case may any aircraft be operated unless there is an appropriate and valid airworthiness certificate issued for that aircraft. The FAA must conduct any inspections necessary to verify the certification procedures listed below, including any other inspections found appropriate for that certification. For amateur-built aircraft, refer to paragraph 147 of this order, and for LSA, refer to paragraphs 123 and 142 of this order.

a. Record Inspection. The FAA representative must do the following:

- Obtain from the applicant a properly executed Form 8130-6 and any other documents required for the certification.						Yes  No

- Obtain from the applicant a program letter which identifies the aircraft, the purpose of the certificate, the area over which the operations are to be conducted, the duration of the program, etc.							Yes  No

- Review the documentation provided by the applicant to determine that the registration requirements of part 47 have been met, and ensure that the aircraft is marked in accordance with part 45.							Yes  No

- Check with AFS-750 to determine if a denial letter exists for the particular aircraft. This may assist the ASI in determining aircraft eligibility.		Yes  No

- Review the aircraft records to determine that any required maintenance, inspections, etc., have been accomplished. Records should be complete and reflect no unapproved design changes.								Yes  No

- Arrange to review any inspection or technical data needed to establish conformity to type design.								Yes  No

- Review the applicant's weight and balance data for accuracy and currency for the aircraft submitted.								Yes  No

- Determine that the aircraft has been flight tested, if required. If it has not been flight tested, issue an appropriate Form 8130-7, for showing compliance with the airworthiness regulations (§§ 21.189(a)(2), 21.185(d) and 91.319(b)). The flight test must be recorded in the aircraft records and certify that the requirements of § 91.319(b) have been met. Flight test time is included as "time-in-service," as defined by part 1.									Yes  No

- Determine that all relevant ADs have been complied with.		Yes  No

NOTE

Each AD contains an applicability statement specifying the product to which it applies. ADs, unless specifically limited, apply to the make and model set forth in the applicability statement regardless of category. The TC and airworthiness certification categories are used to identify the product affected. For further guidance see AC 39-7, Airworthiness Directives for General Aviation Aircraft.

- Establish that all required documentation and records have been provided for the aircraft, that is, an up-to-date approved flight manual, equipment list, and maintenance records and manuals as required by certain airworthiness parts of the CFR.									Yes  No

b. Aircraft Inspection.  The FAA must arrange with the applicant to make the aircraft available for inspection to determine the following:

- The aircraft is eligible by make and model using the TCDS, aircraft specification, or aircraft listing, as applicable.						Yes  No

- The ID plate meets the requirements of § 45.11, as applicable.		Yes  No

- The information on the ID plate is correct, matches the information on Form 8130-6, and is in accordance with § 45.13, as applicable.				Yes  No

- The aircraft nationality and registration marks are in accordance with part 45.												Yes  No

# NOTE

Section 21.182 (a) and (b)(2) requires each aircraft to be identified as described in § 45.11. In addition, if the aircraft previously was registered in the United States, it is acceptable to continue use of the duplicate pink copy of Aeronautical Center Form 8050-1, Aircraft Registration Application, as temporary authority to operate. However, it first must be verified that AFS-750 has received the Aircraft Registration Application as a temporary authority to operate.

- The flight control system operates properly.				Yes  No

- The engine(s), propeller(s), and associated instruments operate in accordance with the manufacturer's instructions.						Yes  No

- The pitot static system and associated instruments operate properly.	Yes  No

- The instruments are marked in accordance with the approved flight manual or any other data used for aircraft involved in a type certification program.	Yes  No

- All modifications have been inspected and recorded, and are in a condition for safe operation.									Yes  No

- An emergency locator transmitter (ELT) is installed, as required in accordance with § 91.207.								Yes  No

c. Certificate Issuance.

(1) If the aircraft meets the requirements for the certification requested, the FAA must-

  - Make an aircraft logbook entry.						Yes  No

  - Issue Form 8130-7.							Yes  No

  - Complete sections V and VIII of Form 8130-6, as appropriate, in accordance with the instructions contained in chapter 8 of this order.				Yes  No

  - Examine, review, and route the certification file in accordance with the instructions contained in chapter 8 of this order.					Yes  No

(2) If the aircraft does not meet the requirements for the certification requested, and the airworthiness certificate is denied, the FAA must-

- Write a letter to the applicant stating the reason(s) for denying the airworthiness certificate.									Yes  No

- Attach a copy of the denial letter to Form 8130-6 and forward to AFS-750 to be made part of the aircraft record.							Yes  No

89. SPECIAL AIRWORTHINESS CERTIFICATES.

c. The duration of amateur-built, exhibition, air racing, light-sport category, and LSA experimental certificates will be unlimited unless the Administrator finds good cause that a specific period should be established. Any other necessary operating limitations will be attached to this form; see paragraphs 126, 144, 153, and 161 of this order.

98. DISPLAY OF MARKS (RESTRICTED).

The FAA must determine that the aircraft displays nationality and registration marks in accordance wìth § 45.21 and that the word "RESTRICTED" is displayed in accordance with § 45.23.									Yes  No