Light Sport Definition

A special airworthiness certificate in the light-sport category is issued to an aircraft that meets the definition of LSA, is manufactured to the applicable consensus standard, and is included in one of the six classes of LSA.  The five classes of the LSA category are:

- Airplanes

- Gliders

- Powered parachutes

- Weight-shift-control aircraft (commonly called trikes)

- Lighter-than-air aircraft (balloons and airships).

NOTE:  Mark one above

Helicopters and powered-lift aircraft are not LSA.  When the aircraft meets the eligibility requirements for certification, it may be issued an airworthiness certificate in the LSA category.  Excluded from the LSA category are gyroplane aircraft (the sixth class), or transitioning ultralight-like vehicles, and kit aircraft, which may receive an experimental purpose for operating LSA as addressed in section 8 of FAA Order 8130.2, Experimental Light-Sport Aircraft Airworthiness Certifications.

Definition. As defined in § 1.1, a light-sport aircraft is an aircraft other than a helicopter or powered-lift that since its original certification has continued to meet the following:

- A maximum takeoff weight of not more than 660 pounds (300 kilograms) for lighter-than-air aircraft; 1,320 pounds (600 kilograms) for aircraft not intended for operation on water; or 1,430 pounds (650 kilograms) for aircraft intended for operation on water.

- A maximum airspeed in level flight with maximum continuous power (VH) of not more than 120 knots calibrated airspeed under standard atmospheric conditions at sea level.

- A maximum never-exceed speed (VNE) of not more than 120 knots calibrated airspeed for a glider.

- A maximum stalling speed or minimum steady flight speed without the use of lift-enhancing devices (VS1) of not more than 45 knots calibrated airspeed at the aircraft's maximum certificated takeoff weight and most critical CG.

- A maximum seating capacity of no more than two persons, including the pilot.

- A single, reciprocating engine, if powered.

- A fixed or ground-adjustable propeller, if a powered aircraft other than a powered glider.

- A fixed or auto-feathering propeller system, if a powered glider.

- A fixed-pitch, semi-rigid, teetering, two-blade rotor system, if a gyroplane.

- A nonpressurized cabin, if equipped with a cabin.

- Fixed landing gear, except for an aircraft intended for operation on water or a glider.

- Fixed or repositionable landing gear, or a hull, for an aircraft intended for operation on water.

- Fixed or retractable landing gear for a glider.

NOTE

Although gyroplane aircraft (commonly known as gyrocopters) are identified in the light-sport aircraft definition of § 1.1, gyroplane aircraft when meeting the criteria may be issued an experimental certificate for the purpose of operating LSA because of the preclusion of § 21.190(a).