| ORDER: | 8300.10 |
| --- | --- |
| APPENDIX: | 3 |
| BULLETIN TYPE: | Flight Standards Handbook Bulletin for |
|  | Airworthiness (HBAW) |
| BULLETIN NUMBER: | HBAW 99-02A |
| BULLETIN TITLE: | Guidance for Issuing Automated Operations Specifications for 14 CFR Part 145 Repair Stations |
| EFFECTIVE DATE: | 03-02-99 |
| AMENDED DATE: TRACKING NUMBER: | 01-28-04 N/A |
| M/MATA Code14 CFRPTRSN/AN/A145N/A APPLICABILITY: |  |

-----------------------------------------------------------------

1.  PURPOSE.  This bulletin provides information and guidance to the Flight Standards Airworthiness aviation safety inspectors (ASI) who have certificate and surveillance responsibility for Title 14 of the Code of Federal Regulations (14 CFR) part 145 repair stations located within and outside the United States.  This bulletin has been developed to supplement existing guidance in the FAA Order 8300.10, Airworthiness Inspector’s Handbook, volume 2, chapters 161, 162, 163, 164, and 165.

2.  BACKGROUND.

A.  The Aircraft Maintenance Division, AFS-300, has incorporated updates and changes to the automated Operations Specifications Subsystem (OPSS).  The OPSS is used in developing and issuing repair station Operations Specifications (OpSpecs), which include revised and new paragraphs to facilitate the

part 145 rule change.  The automated OPSS provides for standardization among all certificate holders.

NOTE:  The term “certificate holder” used in this bulletin includes the holder of a repair station certificate as described in the OpSpecs.

B.  The Civil Air Regulations (CAR) part 52 established requirements for the issuance of repair station certificates, ratings, and OpSpecs, along with operating rules for repair stations.  Today, part 145, section 145.5 sets forth the requirements for repair station OpSpecs.

C.  AFS-300 has determined that within the aviation industry, there is a need to establish and administer safety standards to accommodate the variables allowed for repair stations.  These variables include a wide range of services and the capabilities of each repair station.  The OPSS provides an effective method for standardized OpSpecs for repair stations.

D.  Flight Standards Service developed automated OpSpecs to keep abreast of rapidly advancing technology made in maintenance and alteration of aircraft.  Automated OpSpecs provide computerized access to a standardized format that includes only those authorizations, limitations, standards, and procedures that are applicable to each individual repair station.

NOTE:  The Repair Station OpSpecs (FAA Form 8400-4-1) was replaced by the OPSS.

3.  GUIDANCE.  Issuance of OpSpecs.  The OpSpecs lettering and numbering system used for repair stations is consistent with those in use by other 14 CFR certificate holders.  Part 145 OpSpecs are currently divided into three parts, each of which has an assigned letter designator and contains standard paragraphs.  (See attached samples of unchanged, revised, and new OpSpecs paragraphs; each OpSpec will be identified to reflect its status.)

NOTE:  The OpSpecs are accounted for and controlled in the table of contents and by the signature blocks at the end of each part/paragraph.

A.  Part A OpSpec paragraphs are considered to be general paragraphs and are issued for repair stations located within and outside the United States as follows:

(1)  OpSpec A001, Issuance and Applicability.  OpSpec A001 location, mailing address if different than fixed location, other business names (DBAs), if authorized, and any delegated authorities.

(2)  OpSpec A002, Definitions and Abbreviations.  OpSpec A002 includes the definitions of words or phrases used in the OpSpec paragraphs.  These definitions are not found in other documentation (i.e., 14 CFR, etc.).  Definitions are developed by Washington headquarters and shall not be changed by regional or district offices.  Washington headquarters will add new definitions when it becomes apparent that a definition is needed.

(3)  OpSpec A003, Ratings and Limitations.  OpSpec A003 contains the certificate holder’s authorizations for ratings and limitations.  OpSpec A003 lists the authorized Class Ratings, Limited Ratings, and Limited Specialized Services Ratings held by the certificate holder. This paragraph has been revised to allow the selection of a capability list when a Limited Rating has been issued.  The cells labeled “Manufacturer” and “Make/Model” display a drop-down menu when selected.  The selection of from the accepted “Capability List as Amended” from either cell will auto-fill when selected. The phrase from the accepted “Capability List as Amended” indicates that the certificate holder will be using a current capability list that includes any additions made to the list using the self-evaluation process described in its manual.

(4)  OpSpec A004, Summary of Special Authorizations and Limitations.  This paragraph summarizes special authorizations and/or limitations applicable to the certificate holder.  The OPSS application extracts the specific paragraphs that authorize a specific activity and prints the titles of the paragraphs or equivalent phrases.  When printed, the title (or equivalent phrase) completes the lead-in phrase authorizing the specific activity and reference number of the specific paragraph.

(5)  OpSpec A005, Exemptions.  For the certificate holder to conduct operations under the provisions of an exemption, the exemption number and exemption date (including any remarks or limitations, if applicable), must be listed in this paragraph.

(6)  OpSpec A007, Designated Persons.  This paragraph will list the persons designated by the repair station who can apply for and receive OpSpecs.  They will be listed by name, title, and the part of the OpSpec they are authorized to sign.

(7)  OpSpec A025, Electronic Recordkeeping System and/or Electronic Signature.  This paragraph identifies the approved electronic recordkeeping system used by the certificate holder and identifies its electronic signature procedures.

(8)  OpSpec A060, Joint Aviation Authority Ratings for Repair Stations Located Outside the United States.  This paragraph authorizes repair stations located outside the United States to perform maintenance with Joint Aviation Authority (JAA) ratings where the appropriate form (JAA Form 3) authorizes the scope of work.

(9)  OpSpec A101, Additional Fixed Locations.  This paragraph identifies additional locations (facilities) that collectively form the certificated repair station’s operational base without having to certificate each facility as a stand-alone or satellite repair station.  All additional locations will be under the full control of the primary facility listed in OpSpec A001.  Each separate facility is not required to be completely equipped since tools, equipment, parts, etc. can be transported between facilities.  The Repair Station Manual (RSM) will detail procedures that the repair station must follow during the transport of equipment and parts between facilities, and how it will ensure adequate personnel are available to support the facility while an article is maintained.  Further, using additional fixed locations does not constitute work away from the repair station.

B.  Part B OpSpecs contain special authorizations, limitations, and procedures that may be issued for repair stations located within and outside the United States.

(1)  Currently, Part B only contains OpSpec B050, which applies to repair stations located outside the United States.

(2)  OpSpec B050 provides the certificate holders with authorization to conduct services under contract to a U.S. carrier/14 CFR part 129 operator at a location other than the repair station’s facility.  (Reference Order 8300.10 volume 2, chapter 163.)

C.  Part D OpSpecs contain specific authorizations, limitations, and procedures.

(1)  OpSpec D100, Work to be Performed at a Place Other than the Repair Station’s Fixed Location.  This paragraph is issued when the repair station has been authorized to perform work on either a recurring basis or under special circumstances at a place other than its fixed location by moving facilities, material, equipment, and technical personnel to perform specific maintenance functions.

(a)  The RSM must set forth procedures, which govern how the repair station will perform work on a recurring basis at a place other than the repair station.  OpSpec D100 must list the work authorized and the RSM reference.

(b)  A special circumstance work function cannot be listed in OpSpec D100 due to its unknown origin.  If the RSM has a notification and/or procedural process for a potential special circumstance, then this reference must be listed in OpSpec D100.

(c)  Continuous operations away from the repair station are not authorized by the issuance of this paragraph.

(2)  D107, Line Maintenance Authorization.  This paragraph authorizes the certificate holder to perform line maintenance for certificate holders that conduct operations under 14 CFR parts 121, 125, and 135, and for foreign air carriers or foreign persons operating a U.S.-registered aircraft under part 129.  The certificate holder must be appropriately rated for the maintenance to be performed.  As a minimum, the repair station must have a limited Airframe or Airframe Class Rating to perform inspections and minor maintenance associated with a line maintenance authorization.  A repair station with a limited Airframe rating must state in the limitations cell of OpSpec A003 the work to be performed.  Regardless of the type airframe rating of the repair station, to perform line maintenance, OpSpec A003 must be issued.  The certificate holder must also have the facilities, equipment, trained personnel, and technical data to perform such line maintenance. Table 1 of OpSpec D107 lists the locations where the repair station may perform line maintenance.  Line maintenance operations may be performed on a continuous basis.

4.  ACTION.  ASIs who have certificate management responsibility for part 145 repair stations shall issue the revised and applicable new OpSpecs from the OPSS.  The revised and new OpSpecs will become available paragraphs on the effective date of the revised rule.  Transition to the revised and new OpSpecs must be accomplished within 180 days from the part 145 effective rule date of January 31, 2004.

5.  INQUIRIES.  This bulletin was developed by AFS-340, General Aviation and Repair Station Branch.  Direct any questions regarding this bulletin to AFS-340 at (202) 267-7027.

6.  LOCATION.  The material covered by this bulletin will be incorporated into FAA Order 8300.10, volume 2, chapter 161.  Until the material is incorporated, inspectors should make written reference to this bulletin in the margin of the affected handbook chapter.

/s/ Carol E. Giles for

David E. Cann, Manager

Aircraft Maintenance Division

ATTACHMENTS

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

TABLE OF CONTENTS

PART A

|  |  | HQ CONTROL DATE | EFFECTIVE DATE |  | AMENDMENT NUMBER |
| --- | --- | --- | --- | --- | --- |
| 1 | Issuance and Applicability | 10/20/00 |  | 10/20/00 | 0 |
| 2 | Definitions and Abbreviations | 10/15/00 |  | 10/15/00 | 03b |
| 3 | Ratings and Limitations | 10/15/03 |  | 10/15/03 | 00c |
| 4 | Summary of Special Authorizations and Limitations | 09/23/98 |  | 10/16/98 | 010 |
| 5 | Exemptions | 02/20/98 |  | 10/16/98 | 00a |
| 7 | Designated Persons | 10/15/03 |  | 10/15/03 | 0 |
| 25 | Electronic Recordkeeping System and/or Electronic Signature | 04/04/03 |  | 04/04/03 | 0 |
| 60 | Joint Aviation Authority Ratings for Repair Stations Located Outside the United States | 10/15/03 |  | 10/15/03 | 0 |
| 101 | Additional Fixed Locations | 10/15/03 |  | 10/15/03 | 0 |
| 107 | Line Maintenance Authorization | 10/15/03 |  | 10/15/03 | 0 |

CERTIFICATE NO.:  DCAR0987

DC Repair

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| A001. | Issuance and Applicability | HQ Control: | 10/20/00 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 030 |

UNCHANGED

a.	These operations specifications are issued to TEXT01, a TEXT02  The repair station certificate holder shall conduct operations in accordance with CFR Part 145 and these operations specifications.

The certificate holder’s address:

| Fixed Location: TEXT03 | TEXT04 |
| --- | --- |

b.	The holder of these operations specifications is the holder of Certificate Number TEXT05 and shall hereafter be referred to as the certificate holder.

c. These operations specifications are issued as part of this Repair Station Certificate, and are in effect as of the Date Approval is effective.  This certificate and operations specifications shall remain in effect until TEXT06

TEXT07A TEXT07B

TEXT08

TABL01

Delegated authorities:  TEXT09

TEXT99

1.	Issued by the Federal Aviation Administration.

2.	These Operations Specifications are approved by direction of the Administrator.

3.	Date Approval is effective:	Amendment Number:

4.	I hereby accept and receive the Operations Specifications in this paragraph.

|  |  | Date: |
| --- | --- | --- |

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| A002. | Definitions and Abbreviations | HQ Control: | 10/15/03 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 03b |

REVISED

Unless otherwise defined in these operations specifications, all words, phrases, definitions, and abbreviations have identical meanings to those used in the Title 14 Code of Federal Regulations (CFR) Regulations and Title 49 United States Code as cited in Public Law 103-272, as amended.  Additionally, the definitions listed below are applicable to operations conducted in accordance with these operations specifications.

| Bilateral Aviation Safety Agreement (BASA) | An executive agreement concluded between the United States and a foreign country for the purpose of promoting aviation safety; also known as an Agreement for the Promotion of Aviation Safety. |
| --- | --- |
| Certificate Holder | In these operations specifications the term "certificate holder" shall mean the holder of the repair station certificate described in these operations specifications in Part A paragraph A001 and any of its officers, employees, or agents used in the conduct of operations under this certificate. |
| CFR | Code of Federal Regulations |
| Class rating | As used with respect to the certification, ratings, privileges, and limitation of aircraft within a category having similar operating characteristics. |
| Domestic Repair Station | A certificated repair station located in the United States. |
| Exemption | An authorization that permits an alternate means of compliance with a specific CFR. The exemption must meet the procedural requirements of CFR 14, Part 11. |
| FAA Accountable Manager | A person designated by the certificated repair station who is responsible for and has authority over all repair station operations that are conducted under part 145, including ensuring that the repair station’s personnel follow the regulations and serving as the primary contact with the FAA. |
| FAA Form 8000-4-1 | The FAA Form 8000-4-1 has been replaced with the form contained within this operations specifications application (OPSS) for the generation of the 14 CFR Part 145 Repair Station Operations Specifications. |
| Foreign Repair Station | A certificated repair station located outside of the United States. |
| Geographic Authorization | An approval provided to a foreign repair station to perform maintenance support under contract for a U.S. air carrier, or operator of U.S. - registered aircraft under FAR Part 129, at a location other than the repair station facility. A geographic authorization is issued by the FAA to respond to the need of a U.S. air carrier or FAR Part 129 operator for maintenance at a station where the frequency and scope of that maintenance does not warrant permanently staffing and equipping the station for its accomplishment. |
| JAA | Joint Aviation Authority |
| JAA Accountable Manager | The manager who has corporate authority for ensuring that all maintenance required by the customer can be financed and carried out to the standard required by the JAA full member Authority. |
| Limited Rating | Rating issued to repair stations for the performance of maintenance on particular makes and models of airframes, powerplants, propellers, radios, instruments, accessories, and/or parts. |
| Limited Ratings - Specialized Services | Rating issued for a special maintenance function when the function is performed in accordance with a specification or data acceptable to the Administrator. |
| Line Maintenance | Any unscheduled maintenance resulting from unforeseen events; or scheduled checks that certain servicing and/or inspections that do not require specialized training, equipment, or facilities. |
| Maintenance | The inspection, overhaul, repair, preservation, and the replacement of parts, but excludes preventive maintenance. |
| Maintenance Implementation Procedures (MIP) | Procedures for implementing the provisions of a BASA that apply to maintenance. |
| MOE | Maintenance Organization Exposition - Pertains to Joint Aviation Authority member countries that use an MOE in place of a Repair Station Manual and a Quality Control Manual. |
| Preventive Maintenance | As defined in Appendix A subparagraph (c) of 14 CFR Part 43. |
| QCM | Quality Control Manual |
| RSM | Repair Station Manual |
| Substantial # Maintenance | Any activity involving a C-check (routine airframe maintenance) or greater maintenance; any engine maintenance requiring case separation or teardown; and/or major alterations or major repairs performed on airframes, engines, or propellers. |

TEXT99

1.	Issued by the Federal Aviation Administration.

2.	These Operations Specifications are approved by direction of the Administrator.

3.	Date Approval is effective:	Amendment Number:

4.	I hereby accept and receive the Operations Specifications in this paragraph.

|  |  | Date: |
| --- | --- | --- |

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| A003. | Ratings and Limitations | HQ Control: | 10/15/03 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 00c |

REVISED

The Certificate Holder is authorized the following Ratings and/or Limitations:

Class Ratings

TABL01

TABL02

| TABL05 | TABL04 |
| --- | --- |

| Limited Ratings |  |  |  |
| --- | --- | --- | --- |
| Rating | Manufacturer | Make / Model | Limitations |
| TABL03 | TABL06 | TABL07 | TABL08 |

Note: After a specific limited rating is selected from the rating cell, and  either the Manufacturer or Make/Model cell is selected a drop down menu listing will allow the selection of a Capability List; the verbiage will state from the accepted “See Capability List As Amended”. Tick this selection if the certificate holder chooses to use a capability list.

| Limited Ratings - Specialized Services |  |  |
| --- | --- | --- |
| Rating | Specifications | Limitations |
| TABL11 | TABL12 | TABL13 |

TEXT99

1.	Issued by the Federal Aviation Administration.

2.	These Operations Specifications are approved by direction of the Administrator.

3.	Date Approval is effective:	Amendment Number:

4.	I hereby accept and receive the Operations Specifications in this paragraph.

|  |  | Date: |
| --- | --- | --- |

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| A004. | Summary of Special Authorizations and Limitations | HQ Control: | 9/23/1998 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 010 |

REVISED

a.	The certificate holder, in accordance with the reference paragraphs, is authorized to:

|  | Reference Paragraphs |
| --- | --- |
| Use Exemptions. | A005 |
| Use an approved electronic recordkeeping system and/or electronic signature. | A025 |
| Perform maintenance with JAA ratings where the scope of work is authorized by the appropriate JAA form 3. | A060 |
| Perform maintenance in accordance with foreign repair station geographic authorizations. | B050 |
| Perform work at a place other than the repair station fixed location. | D100 |
| Perform maintenance at additional fixed locations | A101 |
| Perform line maintenance for certificate holders conducting operations under 14 CFR Parts 121, 125, and 135, and for foreign carriers or foreign persons operating a U.S.-registered aircraft in common carriage under 14 CFR Part 129. | D107 |

b.	The certificate holder is not authorized and shall not:

1.	Issued by the Federal Aviation Administration.

2.	These Operations Specifications are approved by direction of the Administrator.

3.	Date Approval is effective:	Amendment Number:

4.	I hereby accept and receive the Operations Specifications in this paragraph.

|  |  | Date: |
| --- | --- | --- |

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| A005. | Exemptions | HQ Control: | 02/20/98 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 00a |

UNCHANGED

The certificate holder is authorized to perform operations in accordance with the provisions, conditions, and/or limitations set forth in the following exemptions listed in the following table.

a.  Exemptions.

| Exemption Number | Date of Expiration | Remarks and/or References |
| --- | --- | --- |
| Exemption Number | Date of Expiration | Exemptions Remarks |

The certificate holder is not authorized to conduct any operations under the provisions of any exemptions.

TEXT99

1.	Issued by the Federal Aviation Administration.

2.	These Operations Specifications are approved by direction of the Administrator.

3.	Date Approval is effective:		Amendment Number:

4.	I hereby accept and receive the Operations Specifications in this paragraph.

|  |  | Date: |
| --- | --- | --- |

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| A007. | Designated Persons | HQ Control: | 10/15/03 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 010 |

REVISED

a.  The personnel listed in the following table are designated to officially apply for and receive operations specifications for the certificate holder indicated below.

| Title | Name | Parts Authorized |
| --- | --- | --- |
| Director of Maintenance | Joe Safeguard | A, B, C, D |
| FAA Accountable Manager | Arnie Marsh | A, B. C.D |
| JAA Accountable Manager | Sara J. Manger | A, B, D |

TEXT99

1.	Issued by the Federal Aviation Administration.

2.	These Operations Specifications are approved by direction of the Administrator.

3.	Date Approval is effective:		Amendment Number:

4.	I hereby accept and receive the Operations Specifications in this paragraph.

|  |  | Date: |
| --- | --- | --- |

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| A025. | Electronic Recordkeeping System and/or Electric Signature | HQ Control | 04/04/03 |
| --- | --- | --- | --- |
|  |  | HQ Revision | 000 |

UNCHANGED

a.	The certificate holder is authorized to use an approved electronic recordkeeping system, described and/or referenced in this paragraph (if none, enter N/A.)

TEXT01

b.	The certificate holder is authorized the use of the following electronic signature procedures (if none, enter N/A):

TEXT02

TEXT99

1.	Issued by the Federal Aviation Administration.

2.	These Operations Specifications are approved by direction of the Administrator.

3.	Date Approval is effective:		Amendment Number:

4.	I hereby accept and receive the Operations Specifications in this paragraph.

|  |  | Date: |
| --- | --- | --- |

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| A060. | Joint Aviation Authority Ratings for Repair Stations located outside the United States | HQ Control: | 10/15/03 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 000 |

REVISED

a.  The certificate holder is authorized the following Joint Aviation Authority (JAA) ratings where the scope of work is authorized by the appropriate JAA Form 3.

- JAA ratings are limited by type and weight of aircraft

- A rating may be issued for base or line maintenance

- Rotors are also listed under components (C-10) and transmissions (C-11)

b.  The Certificate Holder is authorized the following JAA Aircraft Ratings per the scope of work identified on the applicable JAA Form 3:

Select the appropriate JAA aircraft rating and enter the corresponding JAA Form 3 certificate no. for the scope of work authorized:

| JAA AIRCRAFT RATINGS | SCOPE OF WORK AUTHORIZED |
| --- | --- |
| Select A-1 Aeroplanes/Airships (above 5700 kg) | Enter JAA Form 3 Certificate No. for A-1 |
| Select A-2 Aeroplanes/Airships (5700 kg & below) | Enter JAA Form 3 Certificate No. for A-2 |
| Select A-3 Helicopter | Enter JAA Form 3 Certificate No. for A-3 |

c.  The Certificate Holder is authorized the following JAA Engine and APU Ratings per the scope of work identified on the applicable JAA Form 3:

Select the appropriate JAA engine and APU rating and enter the corresponding JAA Form 3 certificate no. for the scope of work authorized:

| JAA ENGINE RATINGS | SCOPE OF WORK AUTHORIZED |
| --- | --- |
| Select B-1 Turbine | Enter JAA Form 3 Certificate No. for Engine Type (B-1) |
| Select B-2 Piston | Enter JAA Form 3 Certificate No. for Engine Manuf. (B-2) |
| Select B-3 APU | Enter JAA Form 3 Certificate No. for APU (B-3) |

d.  The Certificate Holder is authorized the following JAA Ratings for Components other than complete engines or APU per the scope of work identified on the applicable JAA Form 3:

Select the appropriate JAA  rating for components other than complete engines or APU and enter the corresponding JAA Form 3 certificate no. for the scope of work authorized:

| JAA RATINGS FOR COMPONENTS | SCOPE OF WORK AUTHORIZED |
| --- | --- |
| Select C-1 Air Cond & Pres | Enter JAA Form 3 Certificate No. for C-1 |
| Select C-2 Auto Flight | Enter JAA Form 3 Certificate No. for C-2 |
| Select C-3 Comms & Nav | Enter JAA Form 3 Certificate No. for C-3 |
| Select C-4 Doors & Hatches | Enter JAA Form 3 Certificate No. for C-4 |
| Select C-5 Electrical Power | Enter JAA Form 3 Certificate No. for C-5 |
| Select C-6 Equipment | Enter JAA Form 3 Certificate No. for C-6 |
| Select C-7 Engine—APU | Enter JAA Form 3 Certificate No. for C-7 |
| Select C-8 Flight Controls | Enter JAA Form 3 Certificate No. for C-8 |
| Select C-9 Fuel—Airframe | Enter JAA Form 3 Certificate No. for C-9 |
| Select C-10 Helicopter—Rotors | Enter JAA Form 3 Certificate No. for C-10 |
| Select C-11 Helicopter-Transmission | Enter JAA Form 3 Certificate No. for C-11 |
| Select C-12 Hydraulic | Enter JAA Form 3 Certificate No. for C-12 |
| Select C-13 Instruments | Enter JAA Form 3 Certificate No. for C-13 |
| Select C-14 Landing Gear | Enter JAA Form 3 Certificate No. for C-14 |
| Select C-15 Oxygen | Enter JAA Form 3 Certificate No. for C-15 |
| Select C-16 Propellers | Enter JAA Form 3 Certificate No. for C-16 |
| Select C-17 Pneumatic | Enter JAA Form 3 Certificate No. for C-17 |
| Select C-18 Protection | Enter JAA Form 3 Certificate No. for C-18 |
| Select C-19 Windows | Enter JAA Form 3 Certificate No. for C-19 |
| Select C-20 Structural | Enter JAA Form 3 Certificate No. for C-20 |

e.  The Certificate Holder is authorized the following JAA Ratings for specialized services per the scope of work identified on JAA Form 3:

Select the appropriate JAA ratings for specialized services and enter the corresponding JAA Form 3 certificate no. for the scope of work authorized:

| JAA RATINGS FOR SPECIALIZED SERVICES | SCOPE OF WORK AUTHORIZED |
| --- | --- |
| Select D-1 Nondestructive Inspection (NDI) | Enter JAA Form 3 Certificate No. for NDI D-1 |

TEXT99

A004 statement: The certificate holder is authorized to perform maintenance with JAA ratings where the scope of work is authorized by the appropriate JAA Form 3.

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| A101. | Additional Fixed Locations | HQ Control: | 10/15/03 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 000 |

NEW

a.  The certificate holder may perform work at the following additional fixed location(s) listed in Table 1, provided it has the facilities, material, equipment and technical personnel to perform the work authorized:

# Table 1

| Additional Location Address(es) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Address | City | State | Country | Postal Code |
| TABL01 | TABL02 | TABL03 | TABL04 | TABL05 |

b.  This authorizes the certificate holder to use multiple locations to perform its operations under a single certificate.

c.  This authorization does not constitute a geographic authorization or work performed at another location.

d.  The certificate holder may perform continuous operations at the facilities listed in Table 1 above.

e.  Privileges of a line station, as set forth by any JAA line station rating, are authorized at these locations, as applicable.

f.  All the authorizations and exemptions authorized for the certificate holder apply at these locations, as applicable.

TEXT99

A004 statement:  The certificate holder is authorized to perform work, including continuous operations, at additional locations other than at its primary Fixed Location.

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

TABLE OF CONTENTS

PART B

|  |  | HQ CONTROL DATE |  | EFFECTIVE DATE |  | AMENDMENT NUMBER |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | Geographic Authorization for Repair Stations Located Outside the United States |  | 10/15/03 |  | 10/15/03 |  | 0 |

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| B050. | Geographic Authorizations for Repair Stations located outside the United States | HQ Control: | 10/15/03 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 000 |

REVISED

- The certificate holder TEXT01 is authorized to perform maintenance away from the repair station in accordance with the geographic authorizations listed in table 1.  All work is to be performed in accordance with the customer’s approved maintenance document and the procedures contained in the repair station’s Quality Control and Repair Station Manual.

Table 1

| Location | Customer | M/M/S | Contract, as revised | Date | CFR | Maintenance Document |
| --- | --- | --- | --- | --- | --- | --- |
| London, UK | Lloyds of London | A-300-600 ST | IAW the approved maintenance program | 03/10/03 | 121 | Quality Control Manual (QCM) |
| Paris, France | Eiffel Tower Aviation | PA-18-L21A | IAW the approved maintenance program | 05/16/02 | 121 | Repair Station Manual (RSM) |
| Boston, USA | Boston Air | B-747-4F6 | IAW the approved maintenance program | 10/15/01 | 121 | Maintenance Organization Exposition |

TEXT99

1.	Issued by the Federal Aviation Administration.

2.	These Operations Specifications are approved by direction of the Administrator.

3.	Date Approval is effective:	Amendment Number:

4.	I hereby accept and receive the Operations Specifications in this paragraph.

|  |  | Date: |
| --- | --- | --- |

## SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

TABLE OF CONTENTS

PART D

|  |  | HQ CONTROL DATE | EFFECTIVE DATE |  | AMENDMENT NUMBER |  |
| --- | --- | --- | --- | --- | --- | --- |
| 100 | Work to be performed at a place other than the Repair Station Fixed Location | 10/15/03 |  | 10/15/03 |  | 0 |
| 107 | Line Maintenance Authorization | 10/15/03 |  | 1015/03 |  | 0 |

| D100. | Work to be Performed at a place other than the Repair Station Fixed Location | HQ Control: | 10/15/03 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 040 |

REVISED

a.	The certificate holder may perform work at a place other than its

Fixed Location (as listed in paragraph A001 of these operations

specifications) provided it has the facilities, material, equipment

and technical personnel to perform the work authorized in the

following table.

Table 1

| # Work Authorized | Repair Stations Manual Reference | Quality Control Manual References |
| --- | --- | --- |
| TABL01 | TABL02 | TABL03 |

- The certificate holder may not perform continuous operation at a

- facility other than the station’s Fixed Location listed in paragraph A001.

- Line Stations.  Privileges of a line station, as set forth by any

JAA line station rating, are not authorized.

d.  Work may be due to a special circumstance or on a recurring basis.  If on a recurring basis, the repair station must have procedures in its manual.

TEXT99

A004 statement:  The certificate holder is authorized to perform work, excluding continuous operations, at additional locations other than at its primary Fixed Location.

1.	Issued by the Federal Aviation Administration.

2.	These Operations Specifications are approved by direction of the Administrator.

3.	Date Approval is effective:	Amendment Number:

4.	I hereby accept and receive the Operations Specifications in this paragraph.

|  |  | Date: |
| --- | --- | --- |

SAMPLE

U.S. Department

of Transportation

Federal Aviation

Administration

Operations Specifications

| D107. | Line Maintenance Authorization | HQ Control: | 10/15/03 |
| --- | --- | --- | --- |
|  |  | HQ Revision: | 000 |

NEW

a.  The certificate holder is authorized to perform line maintenance for certificate holders conducting operations under 14 CFR Parts 121, 125, and 135, and for foreign air carriers or foreign persons operating a U.S.-registered aircraft in common carriage under CFR Part 129.  Line maintenance may be performed for the listed air carriers at the following location(s) listed in Table 1, provided it has the facilities, equipment, trained personnel, and technical data to perform such line maintenance:

# Table 1

| LOCATION ADDRESS(ES) | Air Carrier |
| --- | --- |
| TABL01 |  |

b.  This line maintenance authorization only applies if the certificate holder performs maintenance functions for operators conducting operations under 14 CFR Parts 121, 125, and 135, and for foreign air carriers or foreign persons operating a U.S.-registered aircraft in common carriage under CFR Part 129.

c.  This line maintenance authorization alone does not constitute a rating. The certificate holder must have a limited airframe rating or airframe class rating to perform inspections and minor maintenance associated with a line maintenance authorization. A repair station with a limited airframe rating must state in the limitations cell of paragraph A003 the work to be performed. Regardless of the type airframe rating of the repair station, this paragraph must be issued for a repair station to perform Line Maintenance.

d.  The certificate holder must perform line maintenance in accordance with the operator’s manual, and approved maintenance program of the Part 121, 125, 135, or 129 operator for which the work is being conducted.

e.  This line maintenance authorization does not constitute a geographic authorization.

f.  The certificate holder may perform continuous operations at the facilities listed in Table 1 above.

g.  Privileges of a line station, as set forth by any JAA line station rating, are authorized at the locations listed in Table 1, as applicable.

h.  All the authorizations and exemptions authorized for the certificate holder apply at these locations, as applicable.

i.  Repair stations must be appropriately rated for the type of work performed.

TEXT99

A004 statement:  The certificate holder is authorized to perform Line Maintenance at the listed locations