RAMP INSPECTION

BALLOON CHECKLIST

NOTE:  This is only a guide and should not be considered FAA approved data for 100 hour or annual inspection checklist.  Consult manufacture data for scope and detail of items included for inspections.

Any questions regarding this check sheet may be directed to Denny Pollard at: stacheair1@sbcglobal.net

INTERPRETATION OF THE TERM “AIRWORTHY”

- The aircraft must conform to its type certificate.

- When aircraft configuration and the components installed are consistent with drawings, specifications, and other data that are part of the type certificate (T/C), and include any supplemental T/C and field approval alterations incorporated into the aircraft.

2)  The aircraft must be in condition for safe operation.

a.  Aircraft relative to wear and deterioration i.e., corrosion, fluid leaks, tire wear, window delimitation/crazing.

NOTE:  If one or both of these conditions are not met, the aircraft would be considered unairworthy.  Order 8130.2F para 9

## Revision date June 26, 2006

# Records Inspection

1.  Registration current 					CFR 91.9	Yes  No

2.  Airworthiness Certificate 					CFR 91.203(a)Yes  No

3.  N number N__________ CFR 43.11, 91.417 and CFR 31.81(b)(2)	Yes  No

4.  Operating Limitation - Found in the Flight Manual 								CFR 91.9, 21.5, 23.1581 and CFR 31.81(b)(1)	Yes  No

5.  Equipment list 				CFR 91.9, 23.29, & CAR-3	Yes  No

6.  Weight and Balance limitations in Flight Manual 									CFR 91.9, 23.23, 23.1519, 23.1581, 31.11-16	Yes  No

7.  Radio Station License ** if required ** only if permanently installed in aircraft.												Yes  No

8.  Last annual inspection completed:  	 CFR 43.11 and 91.417	Yes  No

a.  CFR 65 IA Name ________________ Certificate number ____________

b.  145 Repair Station sign off date Repair Station Number _____________

9.  Last 100 hour inspection completed:  date _____________		Yes  No

a.  145 Repair Station number _________________  CFR 145.3/15/57

b.  A&P sign off date Name _________________ Certification No. _____________

CFR 43.15, 65.85, 91.409	Yes  No

10.  All AD’s current and listed in log books CFR 39 & 91.417		Yes  No

11.  FAA-accepted Instructions for Continued Airworthiness (ICAW for Form 337’s) and FAA-accepted maintenance manuals.												CFR 21.31, 21.50, 23.1529,31.85	Yes  No

12.  Alterations IAW approved STC/TSO/PMA/Field Approval or other FAA data, (337’s) 					CFR 21.97/101/113, 31.82 	Yes  No

# Basket Inspection

1.  Type of fuel tanks Per Type Certificate (T/C) Part number and quantity

a.  Check for DOT Stamp (DOT 4E240) 		CFR 31.45 &46	Yes  No

b.  Check tank fastenings					CFR 43.16	Yes  No

2.  Check for pressure relief valve on top of tank 		CFR 31.46	Yes  No

3.  Basket material wicker/rattan condition Sat. __ UnSat. __ CFR 31.39 	Yes  No

4.  Basket interchange, balloon and heater on envelopes Per T/C 		Yes  No

5.  Type of burner Per T/C Sat. ___ UnSat ____ 		CFR 31.47	Yes  No

6.  Shielding to protect parts adjacent to the burner flame CFR 31.47(b)	Yes  No

7.  Fuel lines hoses and fittings (date)				CFR 31.46	Yes  No

8.  Defective Safety wiring or cotter keys 			CFR 31.37	Yes  No

9.  Batteries condition Sat ___ UnSat ___ ** If installed. **

10.  Instrument and placards are correctly located per T/C 										CFR 91.9, 23.1541-1567,31.85	Yes  No

11.  Data plate installed info matches registration 											CFR 45.11/13, 47.3, CFR 31.81	Yes  No

12.  Oxygen bottle ** If installed ** required Green CFR 23.1441-1453	Yes  No

Hydro AA 5 year HR 3 years retire at 24 years AC43.13-1b para 9-51	Yes  No

13.  Type of Igniter Per the T/C ____________ 				Yes  No

14.  Condition of heater coils					CFR 43.16	Yes  No

15.  Fire Extinguisher ** If Required **  charged? 				Yes  No

16.  Load cables, straight and correct 		CFR 31.27(a)(b)	Yes  No

17.  Vertical load tapes or cords condition 		CFR 31.27(a)(b)	Yes  No

18.  If seat belt installed condition 			CFR 31.63		Yes  No

19.  If position light installed 				CFR 31.65 		Yes  No

20.  Projecting objects must be padded 		CFR 31.59(b)		Yes  No

# Instruments

1.  Altimeter check 	 CFR 91.205/405(d),411, CFR 31.85 Required	Yes  No

2.  Rate of climb indicator installed 			CFR 31.85 Required	Yes  No

3.  Fuel quantity gauge for each tank 	CFR 31.85(b)(1) Required calibrated Yes  No

4.  Envelope temperature indicator gage (calibrated)	CFR 31.85 Required	Yes  No

5.  ATC Transponder ** If installed ** CFR 91.413 date _______ 24 calendar month check.									Yes  No

6.  For captive gas balloons, a compass 		CFR 31.95 Required	Yes  No

Each balloon manufacture specifies in the flight manual what equipment must be on board.

Envelope Inspection

1.  Check Deflation line condition 				CFR 31.55	Yes  No

2.  Check Deflation and Vent systems will it open 		CFR 31.55	Yes  No

3.  Suspension and handling/drop lines condition 		CFR 31.53	Yes  No

4.  Fabric condition:

a.  Skit/Scoop is it burned/missing/ condition 		CFR 31.39	Yes  No

b.  Envelope (fabric) condition no holes exceeding limitations 											CFR 31.21/39 Yes  No

c. Conspicuity must be contrasting color or colors 	CFR 31.83 	Yes  No

5.  Rip Cord end must be colored RED. 			CFR 31.75(c) 	Yes  No

6.  Drag rope must be stiffened 				CFR 31.53	Yes  No

7.  Fabric, seams no tears, on stitching not loose		CFR 43.16	Yes  No

8.  Load tape, webbing, and cords condition			CFR 43.16	Yes  No

9.  Check Deflation panel fabric condition (test)		CFR 43.16	Yes  No

10. Check velcro/springs condition (test)			CFR 43.16	Yes  No

11. Check cable attachments (type) 				CFR 43.16	Yes  No

12. Check suspension cables (type)				CFR 43.16	Yes  No