Field Approvals

1.  What is a field approval?

A field approval is a maintenance performance approval for a major repair or major alteration that is performed by a Flight Standards Service, Aviation Safety Inspector.  The process is used for one serial numbered aircraft in accordance with Order 8300.10, Vol. 2, Chapter 1.  The field approval process has been in use for over 40 years and represents a collaboration between the Flight Standards Service, Aircraft Certification Service, designees, and applicants.  Field approvals are not authorized for aircraft operated under 14 CFR part 121.  Some major alterations are not eligible for field approval and must be approved under another method such as a Supplemental Type Certificate (STC).  These alterations are identified in FAA Order 8300.10, Vol. 2, Chapter 1, Change 16.

2.  What are the steps of the field approval process?

The field approval process is laid out in the following steps:

- The applicant proposes to repair or alter one serial numbered aircraft.

- The applicant must determine that the change is a major alteration or repair per 
14 CFR 1.1 and 14 CFR part 43, Appendix A;

- The change is annotated on a FAA Form 337, Major Repair and Alteration;

- The applicant submits FAA Form 337 annotating the change with the data package to the Flight Standards District Office;

- The Flight Standards District Office may meet to assess the scope, complexity of change in light of 14 CFR 1.1 definitions and 14 CFR part 43, Appendix A;

- The Flight Standards District Office determines that either:

  - The data is adequate and no field approval is required.

  - The Aviation Safety Inspector can sign Block 3 of FAA Form 337 to approve the repair or alteration, or

  - Additional data from the applicant is needed if the original data package is found to be inadequate, or

  - The data needs Aircraft Certification Office review in light of its complexity or adequacy, or

  - The alteration is of a type listed in FAA Orders 8300.10 which exceed the basic scope of a Field Approval and must be processed as an STC.

- If the Aircraft Certification Office reviews the data, they may:

  - Determine that the data package is acceptable as is and can be approved as a Field Approval;

  - Support the field approval with engineering review, advocate additional data or testing, assist with the flight test and Airplane Flight Manual supplements;

  - Recommend that the project should be an Aircraft Certification Office managed Supplemental Type Certification project, and should proceed with the Supplemental Type Certification process.

  - The Inspector approves the repair or alteration by signing block 3 of Form 337.

3.  Who needs to know when a field approval is made?

Owners, operators, and persons who repair or alter aircraft, FAA Flight Standards Inspectors, FAA Aircraft Certification Office Engineers, and DERs need to know when a field approval is made.

4.  Where can I find additional information regarding Field Approvals?

Additional information regarding field approvals can be obtained from the following locations:

14 CFR Part 1, Definitions and Abbreviations

14 CFR sections 43.3, 43.5, 43.7, 43.9 and Appendices A and B.

FAA Order 8300.10, Airworthiness Inspector’s Handbook, Vol. 2, Chapters 1 and 79. Also, Change 16 of this order.

AC 43.13-1, Acceptable Methods, Techniques and Practices Aircraft Inspection and Repair;

AC 43.13-2, Acceptable Methods, Techniques, and Practices Aircraft Alterations;

AC 43-9, Maintenance Records; and

AC 43.9-1, Instructions for Completion of FAA Form 337, Major Repair and Alteration (Airframe, Powerplant, Propeller, or Appliance).

5.  How does an applicant, designee, FAA inspector or FAA engineer determine whether an alteration project can be approved under the field approval process?

The use of the Airworthiness Inspector's Handbook, Order 8300.10, in conjunction with FAA Order 8110.45, use of data approved by Designated Engineering Representatives, enables an applicant, designee, FAA inspector and FAA engineer to identify alteration projects that can be approved under the field approval process.  FAA Order 8300.10 describes those alterations by major product line that could be approved by other means, such as through data, coordinated field approvals, or engineering assistance.  FAA Order 8110.45 addresses performing a major alteration with DER data.

6. If I have DER approved data do I still need a field approval?

Data approved by DERs may, or may not, be sufficient by itself to support a major alteration.  Alterations involving flight manual supplements, airworthiness limitations, ground and flight test plans, ground and flight tests, compliance inspections, modifications to critical structure or life limited parts, instructions for continued airworthiness, special conditions, and equivalent level of safety findings may require data that a DER is not normally authorized to approve.  If the alteration requires approval of data beyond the DER's authority, then additional approval, such as an FAA field approval or STC, is required.

7.  Who determines if I have all the data I need to get a field approval or perform a major alteration?

The person performing the alteration work is responsible for certifying that an alteration is accomplished in accordance with all applicable sections of 14 CFR and FAA - approved data.  An installer who is unsure whether they possess FAA approved data adequate to completely support an alteration should contact his or her Flight Standards District Office (FSDO) for assistance.  A DER supporting an alteration by furnishing approved data should indicate whether the data does, or does not, constitute all the data necessary to substantiate compliance of the alteration with all applicable airworthiness regulations.