Types of Approved Data

Approved data to be used for major repairs and major alterations may be one or more of the following.

- Type Certificate Data Sheets

- Aircraft Specifications

- Supplemental Type Certificates (STC's) (Note: Persons interested in using an STC must have approval in accordance with 49 U.S.C., section 44704, from the holder of the STC prior to it's use.)

- Airworthiness Directives (AD's)

- Manufacturer's FAA Approved Data (DOA)

- Designated Engineering Representative (DER) Approved Data With FAA Form 8110-3, Statement of Compliance (Note: This type of data usually requires additional approval.)

- Designated Alteration Station (DAS) Approved Data

- Appliance Manufacturer's Manuals (Excluding Installation Instructions)

AC 43.13-1, Acceptable Methods, Techniques, and Practices (Aircraft Inspection and Repair), may be used directly as approved data (for repairs only) without further approval only when there is no manufacturer repair or maintenance instructions that address the repair and the user has determined that it is:

•  appropriate to the product being repaired;

•  directly applicable to the repair being made; and

•  not contrary to manufacturer's data.

This data may also be used as a basis to gain FAA data approval for major repairs.

FAA FIELD APPROVAL (FAA FORM 337) issued for duplication of identical aircraft may be used as approved data only when the identical alteration is performed on an aircraft of identical make, model, and series by the original modifier. FAA Form 337's info. approved in 1955 or earlier may be used as approved data.