EXPORT CHECKLIST

Class 1 item

- APPLICATION CFR21.327

- __  Form 8130-1 submitted and properly filled out.

- __  Check chain of ownership from registered owner to exporter.

- __  Required Bill of Sale if registered owner and exporter are different.

- __  Is signature of applicant an authorized representative?

- __  Statement of conformity – Form 8130-9 for new product that has not been manufactured 	under a production certificate.

- REQUIRED DOCUENTS

- __  Must meet requirements for an Airworthiness Certificate. CFR 21.329(a) & (b)

- __  Registration Certificate (Required for Special Flight Permit Only)

- __  Flight Manual(Current – if required) CFR 21.327 (f)(8)

- __  Weight & Balance / Equipment List – Current CRF 21.(f)(2)

- __  Aircraft Maintenance Records as per CFR91.417 CFR 21.(f)(6)

- __  Exporter requirements.  CRF 21.335

- Forward all maintenance records to importer.

- Remove or cause to be removed any temporary installation.

- Secure all foreign entry clearances.

- Ensure Airworthiness & Registration certificates are returned.

# ADDITIONAL REQUIREMENTS

- __  Inspection within proceeding 30 days of application.  CFR 21.329(c)

- Annual inspection only required on export to used aircraft.

- If engine or propeller, item must be new or newly overhauled.  CFR 21.329(d)

- __  Rotorcraft and transport aircraft must be weighed within preceding 12 	months.  CFR 21.327(f)(2)

- __  Special requirements of importing country.  CFR 21.329(f)

- If first of a kind – additional information required.

- __  Required paperwork for any modifications and alterations.

- Do engine, aircraft, and propeller model / serial numbers –

- Match aircraft records.

- Match application

- __  General condition of aircraft.

- __  Conformance to type data sheet.

- __  AD’s current and recorded.  CFR 21.327(f)(4)

- __  Issue Export Certificate.

- __  Complete Export Number Assignment Card- Form8050-72

- __  If shipped-method of preservation, duration and packing.  CFR 21.327(f)(7)

- If overweight ferry system installed, note on Export Certificate under “Exception”.

- On Australian export, note on “Exception” verification of compliance with Australian Air Navigation Order, Section 101.2 deferred until aircraft arrives in Australia.

Note: This is only a guide and should not be considered FAA approved data.