.Inspection authorization: Privileges and limitations CFR Part 65.95

The holder of an inspection authorization may perform:

- Inspect and approve for return to service any aircraft

- Inspect and approve for return to service any related part

- Inspect and approve for return to service any appliance

# NOTE

May approve for return after a major repair or major alteration to it in accordance with CFR Part 43, if the work was done in accordance with technical data approved by the Administrator.

- Perform an annual inspection.

- Perform or supervise a progressive inspection according to CFR 43.13 and 43.15.

- When they exercises the privileges of an inspection authorization the holder shall keep it available for inspection by the aircraft owner, the mechanic submitting the aircraft, repair, or alteration for approval (if any), and shall present it upon the request of the Administrator or an authorized representative of the National Transportation Safety Board, or of any Federal, State, or local law enforcement officer.

If the holder of an inspection authorization MAY NOT exercise the privileges of the authorization:

- Changed their  fixed base of operation without notifying the FAA Flight Standards District Office or International Field Office for the area in which the new base is located, in writing, of the change.

- Except any aircraft maintained in accordance with a continuous airworthiness program under CFR Part121.

Each inspection authorization expires on March 31 of each year.  However, the holder may exercise the privileges of that authorization only while he holds a currently effective mechanic certificate with both a currently effective airframe rating and a currently effective powerplant rating.

An inspection authorization ceases to be effective whenever any of the following occurs:

- The authorization is surrendered, suspended, or revoked.

- The holder no longer has a fixed base of operation.

- The holder no longer has the equipment, facilities, and inspection data required by CFR 65.91(c)(3).

To be eligible for renewal of an inspection authorization for a 1-year period an applicant must present evidence annually, during the month of March, at an FAA Flight Standards District Office, or an International Field Office that the applicant still meets the requirements of §65.91(c) (1) through (4) and must show that:

- Has performed at least one annual inspection for each 90 days that the applicant held the current authority.

- Or has performed inspections of at least two major repairs or major alterations for each 90 days that the applicant held the current authority.

- Or has performed or supervised and approved at least one progressive inspection in accordance with standards prescribed by the Administrator.

- Or has attended and successfully completed a refresher course, acceptable to the Administrator, of not less than 8 hours of instruction during the 12-month period preceding the application for renewal.

- Or has passed on oral test by an FAA inspector to determine that the applicant's knowledge of applicable regulations and standards is current.