Enforcement of Manufacturer’s Data

There exists a difference of opinion among field inspectors concerning the manner in which manufacturer’s maintenance manuals material, including service letters and service bulletins, could be enforced by the FAA.  CFR 43 Section 43.13 requires all persons to use methods, techniques, and practices acceptable to the Administrator while performing aircraft maintenance.  The manufacturer’s maintenance manual, service bulletins, and service letters have always been regarded as a source of acceptable data for complying with CFR 43.13(a) and (b); however, such acceptability does not, in itself, impose an enforcement or mandatory compliance requirement.

The office of the Chief Counsel has advised that the provisions of the manufacturer’s manual, letters, and bulletins, with relation to specific inspection procedures, may be enforceable by means of an airworthiness directive (AD) or other specific rule.  In addition, manufacturer’s data contained in a type certificate data sheet (TCDS) or specification may be enforceable under “CFR 43.13.

An example of a rule that establishes manufacturer’s maintenance inspection criteria as mandatory and enforceable would be CFR 43.15(b), wherein certain inspection criteria for rotorcraft must be performed in accordance with the maintenance manual of the manufacture concerned.  CFR 43.15(c)(2) and 43.16 are other specific examples of regulator requirements that make manufacturer’s procedures mandatory.

The type certificate data sheet and specifications contains conditions, limitations and terms pertinent to the issuance of a particular type certificate.  These limitations are issued under authority of Section 603(a)(2) of the FA Act.  The data sheet or specifications are themselves a part of the type certificate per CFR 21.41 and may require adherence to manufacturer’s maintenance manuals material.  CFR 43.13, in pertinent part, requires that each person maintaining or altering, or performing preventive maintenance, shall use methods, techniques, and practices acceptable to the Administrator and do that work in such a manner and use materials of such quality, that the condition of the aircraft, airframe, aircraft engine, propeller, or appliance worked on will be at least equal (not better than) to its original or properly altered condition.  In general, this means that persons performing maintenance are obligated to establish airworthiness by compliance with the condition and limitations appearing o the type certificate date sheets or specifications.  Thus, enforcement of CFR 43.13(a) and /or (b) is quite possible and feasible against maintenance personnel for noncompliance with the data sheets.

Always check the data sheets notes.  The language on some dates sheets is subject to close evaluation to determine whether or not it is mandatory or informative.  A note that typically reads, “maintenance information may be found in the xyz manual” is informative and not mandatory.  Thus, the notes on data sheet must be carefully read to determine their true impact.

Placards and life-limited parts during inspection require inspecting.  Non-adherence to life limits or failure to follow placarding instructions specified in type certificate date sheets are typical examples of where sound enforcement action can be taken against maintenance entities, under CFR 43.13(a) and (b).

While mechanics are subject to CFR 43 and are expected to establish compliance with type certificate data sheets, owner/operators are not expected to fully knowledgeable of data sheet requirements.  Thus, upon discovering non-adherence to the type certificate date sheet, enforcement action should be directed against the airworthiness certificate of the aircraft.  Exceeding life limits of parts or not maintaining the aircraft according to a specific required mandated are examples of causes for action against the airworthiness certificate.

It should be noted that certain operators are given notice to comply with type certificate data sheets through CFR 91.403.  Failure of an owner/operator to comply after being duly notified, in the regulations, is cause for action against the individual owner/operator.  Action against the airworthiness certificate is also possible under this condition.

To sum up, compliance with manufacturer’s maintenance instructions is required when:

- Made mandatory by an AD or other specific rule within the CFR’s

- Made mandatory by a type certificate data sheet.  Only maintenance people are subject to this requirement since they have been advised to comply through CFR 43; however caution must be exercised since an equivalent procedure could be found to be acceptable at an enforcement hearing.  The FAA case should show that and equivalent level of safety was not attained.  Non-compliance with life limits, placarding instructions, or component configurations specified on a data sheet is always acceptable grounds for initiation enforcement action.