Typical Maintenance entry for 100hr/annual inspection

Reference:  FAA-G-8082-11 IA KNOWLEDGE TEST INFORMATION

FIGURE 9.-Example of a record entry for an annual inspection in which the aircraft was found to be unairworthy.

March 30, 1998

Total Aircraft Time 1853.00 Hours

Tach Reading 975.80

I certify that this aircraft has been inspected in accordance with an annual inspection and a list of discrepancies and unairworthy items dated March 30, 1998, have been provided for the aircraft owner.

Joseph P Kline

A&P 123456789 IA

FIGURE 10.-Example of a discrepancy list to be provided to an aircraft owner when reporting an aircraft with unairworthy items after completing  an annual inspection.

Academy Aviation

Hangar 4

North Philadelphia Airport

Philadelphia, PA 19114

Mr. Morris Mecca

1450 W. Cheltenham Ave.

Philadelphia, PA 19125

Dear Mr. McCall:

This is to certify that on March 30, 1998, I completed an annual inspection on your aircraft, Condor 191B, S/N 3945, N1234, and found the following unairworthy items:

1.  Compression in No. 3 cylinder read 30 over 80, which is below the manufacturer's recommended

limits.

2.  The muffler has a broken baffle plate which is blocking the engine exhaust outlet.

3.  There is a 6-inch crack on bottom of left wing just aft of main landing gear attach point.

Jospeh P. Kline

A&P 123456789 IA