CONDITION INSPECTION

Stolp Star Duster

Web:  http://www.stacheair.com

Acroduster Too SA750

A fully aerobatic, two place design, stressed to +9, -9 G's.  It has modified symmetrical wings and is capable of matching aerobatic performance with any two seat airplane now flying.  Construction follows traditional biplane practices with steel tube structure and spruce wing spars.  Ribs are plywood with cap strips.

Starduster SA100

Is not intended to be an aerobatic airplane.

Starduster Too

Designed as a reasonably sized, two place, open sport biplane.  Not intended for aerobatic flight..  The main structure is built of 4130 steel tubing and sheet stock and has no machined fittings or other complicated bends.  The wings have spruce spars and plywood ribs.  Design stressed to +6, -6 G's.

Starlet SA500

All basic structure except the wing is made of 4130 steel tubing. The wing uses spruce spars and plywood ribs with capstrips.

Super Starduster

Newer design, single place, high performance sport biplane.

V-Star SA900

Biplane that is easy to build.  Light wing loading insures slow landing speed and short takeoff roll.. Welded steel fuselage with spruce and plywood wings. Parts such as gas tanks, nose cowls, wheel pants, and windshields are available ready to install.  Structure stressed to +9, -9 G's

### Airman Records

- Verify A&P or Repairman certificate number and picture ID	CFR 65.91 	Yes  No  N/A

- Verify Owner/Pilot certificate number and picture ID		CFR 61.3 	Yes  No  N/A

Records

- Registration current and in aircraft				CFR 91.9	Yes  No  N/A

- Airworthiness Certificate in aircraft dated and signed	CFR 91.203(a) & 21.175	Yes  No  N/A

- Operation Limitations in aircraft dated and signed	CFR 91.203(a) & 21.175	Yes  No  N/A

- Condition Inspection Total time________		 CFR 43.11 & 91.417 	Yes  No  N/A

- All AD’s, recorded in maintenance records and check recurring AD’s

- Airframe					CFR 39 & 91.417	Yes  No  N/A

- Engine						CFR 39 & 91.417	Yes  No  N/A

- Propeller					CFR 39 & 91.417	Yes  No  N/A

- Appliances  	Check for a list of items		CFR 39 & 91.417	Yes  No  N/A

- Last Condition inspection completed: date ___________________________	CFR 43.11 & 91.417

- CFR 65.91 A&P/Repairman Name ________________ Certification No.________________________

- 145 Repair station sign off date  Repair Station number ______________________________________

- Status of Life Limited parts Effective April 15, 2002		CFR 43.10	Yes  No  N/A

- Current weight and balance report 		CFR 23.23, 23.1519, 23.1581, 91.9	Yes  No

- Identification data plate secured to aircraft fuselage exterior 	CFR45.11	Yes  No  N/A

- Maintenance Records for each Engine, Propeller, Airframe and Appliances inaccordance with 									CFR 91.417 	Yes  No  N/A

- ATC transponder date ___ 24 calendar months check. 		CFR 91.413	Yes  No  N/A

- ELT TSO-C91a /TS0-C126 every 12 months			CFR 91.207(d)	Yes  No  N/A

- Altimeter test every 24 months.				CFR 91.411	Yes  No  N/A

### Cockpit Inspection

- Instrument & placards are correctly located 			CFR 91.31	Yes  No  N/A

- INOP placards 						CFR 91.213	Yes  No  N/A

- INOP instruments disabled or removed by an A&P		CFR 91.213	Yes  No  N/A

- Equipment list up-dated					CFR 91.213	Yes  No  N/A

- Maintenance record entry					CFR 91.213	Yes  No  N/A

- Weight and balance record updated				CFR 91.9	Yes  No  N/A

- Vacuum indicating system, Life limited pump			CFR 23.1301	Yes  No  N/A

- Compass card Yes  No    Readable 		CFR 23.1547 & 25.1547	Yes  No  N/A

- Type of clock installed original analog or digital working 	AC 20-94	Yes  No  N/A

- Nav Radio P/N 1 _________, P/N 2 ________ matches equipment list		Yes  No  N/A

- Cockpit fuel smell 						CFR 23.863	Yes  No  N/A

- Data plate information matches registration 		CFR 45.11/13 & 47.3	Yes  No  N/A

- Instrument filter covers installed 						Yes  No  N/A

- Yoke chain safety wired, loose or corrosion present		CFR 43.13	Yes  No  N/A

- Cockpit control knob shape correct type				CFR 23.781	Yes  No  N/A

- Flap, gear, knobs installed 					CFR 23.781	Yes  No  N/A

- Electrical wiring more than ½ inch slack 		AC 43.13-1B para 11-118	Yes  No  N/A

- Electrical wiring clamps/marking as required 		AC 43.13-1B sec. 11	Yes  No  N/A

- Tie-wraps items of mass in the cabin 				CFR 23.561	Yes  No  N/A

25) Loose wires under dash not clamped 				CFR 23.1351	Yes  No  N/A

26) Fuel selector moves to all positions and placarded 		CFR 23.951	Yes  No  N/A

- Brake master cylinder leaking R/H  L/H  			CRF 43.13	Yes  No  N/A

- Thoroughly clean the aircraft and aircraft engine 			CFR 43App D	Yes  No  N/A

- General, uncleanness and loose equipment that might foul the controls, apparent and obvious defects and insecurity of attachment.					CRF 43App D	Yes  No  N/A

- Stall warning system horn works				CFR 23.1323	Yes  No  N/A

- Windshield and windows conditions				CFR 23.775	Yes  No  N/A

- Windscreen clear  scratches  cracks  crazing  										Reference CFR23.775 AC 43.13-1b and MIL-P-5425 	Yes  No  N/A

- Windows stopped drilled, cracked, or crazed 	CFR 23.775, and	AC 43.13-1B Chapter 3 para3-18(a)(2),and  MIL-P-5425 			Yes  No  N/A

# Instruments						CFR 91.205

## Flight Instruments minimum required

- An Airspeed indicator					CFR 23.1303	Yes  No  N/A

- An Altimeter indicator					CFR 23.1303	Yes  No  N/A

- Direction indicator						CFR 23.1303	Yes  No  N/A

Minimum Instruments Required Visual-flight rules (day)

- Air speed indicator						CFR 91.205(b)	Yes  No  N/A

- Altimeter							CFR 91.205(b)	Yes  No  N/A

- Magnetic direction indicator					CFR 91.205(b)	Yes  No  N/A

- Tachometer for each engine					CFR 91.205(b)	Yes  No  N/A

- Oil pressure gauge for each engine using pressure system		CFR 91.205(b)	Yes  No  N/A

- Manifold pressure gauge for each altitude engine		CFR 91.205(b)	Yes  No  N/A

- Fuel gauge indicating the quantity of fuel in each tank		CFR 91.205(b)	Yes  No  N/A

- Landing gear position indicator, if retractable 			CFR 91.205(b)	Yes  No  N/A

Minimum Instrument for Instrument flight rules

- Two-way radio communications and navigation equipment	CFR 91.205(c)	Yes  No  N/A

- Gyroscopic rate-of-turn indicator				CFR 91.205(c)	Yes  No  N/A

- Slip-skid indicator						CFR 91.205(c)	Yes  No  N/A

- Sensitive altimeter adjustable for barometric pressure		CFR 91.205(c)	Yes  No  N/A

- Clock displaying hours, minutes, and seconds with sweeping second hand pointer or digital presentation									CFR 91.205(c)	Yes  No  N/A

- Generator or alternator of adequate capacity			CFR 91.205(c)	Yes  No  N/A

- Gyroscopic pitch and bank indicator (artificial horizon)		CFR 91.205(c)	Yes  No  N/A

- Gyroscopic direction indicator (directional gyro)			CFR 91.205(c)	Yes  No  N/A

Powerplant Instrument (all aircraft)

- Fuel quantity indicator per tank				CFR 23.1305(a)	Yes  No  N/A

- Oil pressure indicator for each engine				CFR 23.1305(a)	Yes  No  N/A

- Oil temperature indicator for each engine			CFR 23.1305(a)	Yes  No  N/A

- Oil quantity measuring device for each engine			CFR 23.1305(a)	Yes  No  N/A

- Fire warning indicator if required by				CFR 23.1205	Yes  No  N/A

- Tachometer indicator for each engine				CFR 23.1305(b)	Yes  No  N/A

- Cylinder head indicator for each engine				CFR 23.1305(b)	Yes  No  N/A

- Manifold pressure indicator for each engine and for each engine with an controllable pitch propeller										CFR 23.1305(b)	Yes  No  N/A

Instrument Arrangement					CFR 23.1321

- Altitude on panel top center position				CFR 23.1321	Yes  No  N/A

- Airspeed indicator adjacent to and directly to left of the instrument in the top center position.											CFR 23.1321	Yes  No  N/A

- Altitude indicator adjacent to and directly to right of the instrument in the top center position.											CFR 23.1321	Yes  No  N/A

- Magnetic direction indicator placard must state whether the calibration was made with radio receivers on or off								CFR 23.1547	Yes  No  N/A

- Magnetic direction indicator more than 10 degrees off		CFR 23.1547	Yes  No  N/A

Instrument Markings					CFR 23.1543

- Marking on cover glass must be in alignment with face		CFR 23.1543	Yes  No  N/A

- Each arc and line must be clearly visible to pilot			CFR 23.1543	Yes  No  N/A

- All related instruments must be calibrated in compatible units	CFR 23.1543	Yes  No  N/A

Electrical System						CFR 23.1367

- Switches

- Able to carry rated current					CFR 23.1367	Yes  No  N/A

- Enough distance or insulating material between current carrying parts and the housing so that vibration will not cause shorting					CFR 23.1367	Yes  No  N/A

- Labeled as to operation and circuit controlled		CFR 23.1367	Yes  No  N/A

- Circuit Breakers/Fuses

- Circuit protection					CFR 23.1357	Yes  No  N/A

- Each resettable circuit trip free cannot be overridden 		CFR 23.1357 	Yes  No  N/A

- Fuses readily available and spare				CFR 23.1357	Yes  No  N/A

- Breakers labeled and rating 				CFR 23.1357	Yes  No  N/A

- Master Switch

- Wired to disconnect each electrical power source from the distribution systems											CFR 23.1361	Yes  No  N/A

- Switch is easily discernible and accessible to crew		CFR 23.1361	Yes  No  N/A

# Equipment / Furnishings

- Batteries for proper installation, & charging	 		CFR 23.1353	Yes  No  N/A

- Battery vented overboard 		CFR 23.1353 & AC 43.13-1B PARA 11-22	Yes  No  N/A

- Battery NiCad gage for thermal run away			CFR 23.1353	Yes  No  N/A

- Thermal/Noise insulation					CFR 23.853	Yes  No  N/A

- Relief tube, corrosion areas					CFR 43.13	Yes  No  N/A

- New interior material Certs.		 CAR-3.388 and CFR 23.853/25.853	Yes  No  N/A

- Interior replaced Yes  No  New  Have Burn Certifications 		Yes  No  N/A

- Seat covers, sheep skin material Certs. 		CAR-3.388 and CFR 23.853	Yes  No  N/A

- Worn seat material CFR 25.853 for a CAR-3.388 aircraft Part 91 & 23.853, AC 43.13-1B Para 9-61												Yes  No  N/A

- Seat back locks broke 					CFR 23.785	Yes  No  N/A

- Seat rails holes elongated Yes  No  Check AD. 		CFR 39		Yes  No  N/A

- Seat belts for stitching, cuts, or frayed 			CFR 91.107 & 23.785	Yes  No  N/A

- Seat belts proper storage and marking 			TSO-22 &  23.785	Yes  No  N/A

- Seat Belts Plastic locking ring missing, FWD  AFT  	CFR 45.14	Yes  No  N/A

- Seat belt secured when not is use				CFR 23.785(d)	Yes  No  N/A

- TSO C-22 marking on seat belts 		CFR 45.15 CFR 91.205(b)(13,14) 	Yes  No  N/A

- Shoulder harness required after July 18, 1978		           CFR 23.785(g)(1)	Yes  No  N/A

- Shoulder harness required after Sept. 16, 1992(PAX) helicopter		Yes  No  N/A

- Glare shield painted flat black					CFR 23.773	Yes  No  N/A

##### Misc. Fuselage

- Corrosion on antenna’s					CFR 23.609	Yes  No  N/A

- Fabric covered aircraft condition good  poor  	AC 43.13-1B Chapter 2	Yes  No  N/A

- Emergency exit placards 					CFR 23.1557(d)	Yes  No  N/A

- Condition of paint, is corrosion present NOT allowed		CFR 23.609	Yes  No  N/A

- Antenna installation doubles per 		AC43.13 2A, CFR 23.571/572	Yes  No  N/A

### Aircraft exterior inspection

- Nationality and registration marks per CFR 45.29 Check 3 inch marking  per date Jan. 1,1983 and repaint. Over 30 years 2 or12 inch 					CFR 45.22(b)	Yes  No  N/A

- Rotating beacon installation proper doubler Required after 08-11-71 CFR 23.1401	Yes  No  N/A

- Anticollision light system installed 		CFR 91.209(b) / CFR 23.1401	Yes  No  N/A

- AFT nav light  proper color white 			CFR 23.1385-1399	Yes  No  N/A

- Panel seams miss match, extra paint in front of static port	CFR 23.1325	Yes  No  N/A

- Static ports paint in hole or other things L/H  R/H  		CFR 23.1325	Yes  No  N/A

- Static vent painted over			L/H  R/H  	CFR 23.1325	Yes  No  N/A

- Pitot tube worn around hole Yes  No  Not plugged 				Yes  No  N/A

- a. Air Speed Last inspection date _____________		CFR 23.1325	Yes  No  N/A

- Static wicks missing

- Right wing						CFR 23.867	Yes  No  N/A

- Left wing						CFR 23.867	Yes  No  N/A

- Right elevator						CFR 23.867	Yes  No  N/A

- Left elevator						CFR 23.867	Yes  No  N/A

- Rudder							CFR 23.867	Yes  No  N/A

### Landing Gear

- Fairing cracked ,  Hardware missing  .		     CFR 23.607/1193	Yes  No  N/A

- Tire service Main ____ N/T ____				CFR 23.733	Yes  No  N/A

- Tires condition wear, cuts, or weather cracking L/H   R/H   TLG  												  AC 43.13 para 9-14	Yes  No  N/A

- Brake pads worn. L/H  R/H  0.100 inch thickness minimum CFR 23.735	Yes  No  N/A

- Brake lines condition, frayed, corrosion on fittings L/H  R/H 	CFR 23.735	Yes  No  N/A

- Brake rotor corrosion, warped, or under size L/H  RH 	CFR 23.735	Yes  No  N/A

- MLG strut / axle / torque links L/H condition			CFR 23.721 	Yes  No  N/A

- MLG strut / axle / torque links R/H condition			CFR 23.721 	Yes  No  N/A

- Landing gear fairing missing, check equipment list and W&B 	CFR 21.29	Yes  No  N/A

- Main gear trunon strut bushings for wear L/H  R/H 	CFR 43.13	Yes  No  N/A

- Lube type of grease used per manufacture ________________	CFR 43.16	Yes  No  N/A

- Repack wheel bearing					CRR 43 Appendix D	Yes  No  N/A

### Flight Control/Wing

- Wing attach fittings for cracks, elongated bolt holes (right)	L/H CFR 23.572 	Yes  No  N/A

- Wing attach fittings for cracks, elongated bolt holes (left)		R/H CFR 23.572 	Yes  No  N/A

- Wing L/H dents, cracks, Loose rivets, Corrosion, nav light red 	CFR 23.603/1385Yes  No  N/A

- L/H wing Fuel vent direction check AD’s FWD  AFT 	CFR 23.975	Yes  No  N/A

- Wing R/H dents, cracks, Loose rivets, Corrosion, nav light green CFR 23.603/1385 Yes  No  N/A

- R/H Fuel vent direction  			FWD  AFT  CFR 23.975	Yes  No  N/A

- Flaps L/H cracks ______, Loose hardware _______, Properly installed _________.	 Stop drill cracks 									CFR 23.655/697	Yes  No  N/A

- Flaps R/H cracks ______, Loose hardware _______, Properly installed _________.	Stop drill cracks 										CFR 23.655/697	Yes  No  N/A

- Aileron R/H cracks ___, Loose hardware ___, Properly installed 			Yes  No  N/A

- Cable rigging loose Annual/100 hour inspection 		CFR 23.655/685/689	Yes  No  N/A

- Aileron  L/H cracks ___, Loose hardware ____, Properly installed 		Yes  No  N/A

- Cable rigging loose Annual/100 hour inspection		CFR 23.655/685/689	Yes  No  N/A

- Rudder moves up & down, bearing loose, cracks, repairs		CFR 23.685	Yes  No  N/A

- Rudder / trim tab attach fittings	 condition, loose		CFR 23.685 	Yes  No  N/A

- Lubrication, systems lube per manufactures recommendations	CFR 43.13	Yes  No  N/A

- Electrical bonding straps broken or frayed			CFR 23.867	Yes  No  N/A

- Horizontal Stab L/H cracks , Hardware installation and safetied , Stops  												CFR 23.675	Yes  No  N/A

- Horizontal Stab R/H cracks  Hardware installation and safetied  Stops 												CFR 23.675	Yes  No  N/A

- Elevator / trim tab attach fittings L/H				CFR 23.572 	Yes  No  N/A

- Elevator / trim tab attach fittings R/H				CFR 23.572 	Yes  No  N/A

- Elevator trim / servo tab structure	 L/H			CFR 23.572 	Yes  No  N/A

- Elevator trim / servo tab structure	 R/H			CFR 23.572 	Yes  No  N/A

- Elevator trim control system rigging				CFR 23.659 	Yes  No  N/A

- Control surface attach fittings condition			CFR 23.572 	Yes  No  N/A

- Control surface balancing all primary controls after repair or paint	CFR 23.659 	Yes  No  N/A

- Flight Control Surface Travels / Cable Tension			CFR 23.143 	Yes  No  N/A

- Gust lock, condition, worn holes				CFR 23.572 	Yes  No  N/A

- Flight control pulleys worn, broken, or frozen up		CFR 23.689 	Yes  No  N/A

- Flight control cables broken strands/rust Reference AC43.13	CFR 23.689 	Yes  No  N/A

- Flight control Surface Travel/Cable Tension			CFR 23.391 to 23.459

###### Travel Spec Actual	Tension Spec		Actual

Control Column		________________	___________		______

## Aileron			________________	___________		______

Aileron Trim Tab(s)	________________	___________		______

Rudder			________________	___________		______

Rudder Trim Tab		________________	___________		______

Elevator/Stabilator	________________	___________		______

Elev / Stab Tab(s)	________________	___________		______

Flap			________________	___________		______

Moveable Stabilizer	________________	___________		______

Rudder Pedals		________________	___________		______

### Engine Inspection

Reference CFR 23 Subpart E Powerplant

- Engine Data plate installed					CFR 45.13	Yes  No  N/A

- Instruments CFR 23.1305 instruments 						Yes  No  N/A

- a.  Engine         						CFR 23.1301	Yes  No  N/A

- b.  Accessories 						CFR 23.1301 	Yes  No  N/A

- Engine cowl loose/missing hardware Location _____ 		CFR 23.1193	Yes  No  N/A

- Firewall bent, cracked, or missing fasteners 			CFR 23.1191	Yes  No  N/A

- Firewall wire and hose grommets condition			CFR 23.1191(c)	Yes  No  N/A

- Firewall has corrosion 					CFR 23.1191(e)	Yes  No  N/A

- Engine mount structure for cracks, dents, etc. 			CFR 23.23	Yes  No  N/A

- Engine shock mount cracks, worn, hardware condition 		CFR 33.33	Yes  No  N/A

- Flex tubing condition weather cracking, worn, etc. 		CFR 23.1183	Yes  No  N/A

- Engine oil leaking Location ______________________		CFR 23.1183	Yes  No  N/A

- Air Filter dirty/foreign particles				CFR 23.1107	Yes  No  N/A

- Condition of baffle seals and installation Good  Poor  Substituted type of baffle material    								CFR 33.15/17/21& CFR 23.1043	Yes  No  N/A

- Wire chafing, fuel lines, no wires clamped under them 		AC 43.13-1B 	Yes  No  N/A

- Electrical wire Slack between supports Max1/2 inch 	AC 43.13-1B fig.11.9	Yes  No  N/A

- Engine/Electric fuel pump condition wires, mounting Good/worn 	CFR 23		Yes  No  N/A

- Ignition harness condition Good ____ Worn ____		CFR 23		Yes  No  N/A

- Clean and gap spark plugs per engine manufactures recommendations 		Yes  No  N/A

- Starter ring broken teeth 					CFR 23		Yes  No  N/A

- Alternator/generator drive belts condition worn, cracked, broke 	CFR 23		Yes  No  N/A

- Cracked cylinders fins, rocker cover leaking 1__ 2__ 3 __ 4__ 5__ 6__		Yes  No  N/A

- Exhaust stacks cracks, defects, installation 1__ 2 __ 3__ 4__ 5__ 6__ 											CFR 23. 1121 and CFR 33.21	Yes  No  N/A

- Muffler cracked, location _______,Recurring AD’s		CFR 23.1121	Yes  No  N/A

- Muffler leak test to 2 psi internal pressure 	AC 43.13-1b para 8-49 (d)	Yes  No  N/A

- Preheater shroud condition cracks, vibration			CFR 23.1101	Yes  No  N/A

- Carburetor heat box condition of holes 				CFR 23.		Yes  No  N/A

- a.  Proper hardware screws and nuts				CFR 23.		Yes  No  N/A

- Engine controls properly safetying  travel  AC 43.13-1B para 7-122 thru 127	Yes  No  N/A

- Engine case nuts torqued and right side up 	CFR 43 Appendix D (d)(2) 	Yes  No  N/A

- Cowl flap control 						CFR 23.1047	Yes  No  N/A

- Alternate/ Ram air / Carb heat					CFR 23.1093	Yes  No  N/A

- Turbo Waste gate (Non automatic) 				CFR 23.1091	Yes  No  N/A

- Crankcase for cracks, leaks and security of seam bolts 		CFR 33.		Yes  No  N/A

- Vacuum pump lines, clamps condition Good  Worn 	CFR 23		Yes  No  N/A

- Supercharger overall condition Good  Bad 		CFR 23.1109	Yes  No  N/A

- Oil filter opening placard 				       CFR 23.1557(c)(2)	Yes  No  N/A

- Metal in oil filter core paper					CFR 23.1019	Yes  No  N/A

- Electrical wiring cracked, burned , broken 			CRR 23.1163(b)	Yes  No  N/A

- Oil drain plug/valve condition and positive locking 		CFR 23.1021	Yes  No  N/A

- Oil radiator supporting structure for security			CFR 23.1023	Yes  No  N/A

- Oil tanks condition and free of vibration 			CFR 23.1013	Yes  No  N/A

- Engine mounts for corrosion, cracks NONE Allowed		CFR 23.363	Yes  No  N/A

- Hose inspection/replacement manufacture limits	 		CFR 43.10	Yes  No  N/A

- Retorque cylinder base nuts and case half per manufacture recommendations	Yes  No  N/A

- Crank shaft flange cracked							Yes  No  N/A

- Differential Compression Test ,80psi /60 psi cylinder 									CFR 43 Appendix D and AC 43.13-1B paragraph 8-14		Yes  No  N/A

1 ____3 ____4___ 5___6___  If 25% difference check cylinder for problems.

##### Fuel System

- Injection fuel lines Recurring AD if required every 100 hours	CFR 39		Yes  No  N/A

- Fuel bowl leaking 						CFR 23.999	Yes  No  N/A

- Fuel quantity sensor / transmitter condition			CFR 23.955	Yes  No  N/A

- Fuel boost / Aux. pump(s) bypass condition			CFR 23.955	Yes  No  N/A

- Fuel lines vibration/clamped		CFR 23.993 & AC 43.13-1B para 8-31	Yes  No  N/A

- Carburetor security, throttle arm/bushing loose 			CFR 23.994	Yes  No  N/A

- Mixture control linkage condition				CFR 23.1147	Yes  No  N/A

- Throttle control binding condition				CFR 23.1143	Yes  No  N/A

- Engine primer leaking					CFR23.1141	Yes  No  N/A

- Induction System Screens condition				CFR 23.1107	Yes  No  N/A

- Fuel pump condition and AD requirement			CFR 23.991	Yes  No  N/A

- Fuel system lines and fittings conditions			CFR 23.993	Yes  No  N/A

- Fuel system drains, lock shut and drains properly		CFR 23.999	Yes  No  N/A

- Clean and inspect fuel tank strainer condition			CFR 23.977	Yes  No  N/A

- Filler cap must have electrical bonding (chain on cap)		CFR 23.973	Yes  No  N/A

- Fuel placards L/H wing Yes  No  R/H wing		CFR 23.1557(c)	Yes  No  N/A

- Fuel tank caps seal condition, tight seal				CFR 23.973(c)	Yes  No  N/A

- Fuel tank sump drained of water				CFR23.971	Yes  No  N/A

- Fuel tank condition for cracks, vibration, leaks			CFR 23.963	Yes  No  N/A

- Fuel line and hose condition left/right side			CFR 23.993	Yes  No  N/A

- Fuel drain valve positive locking				CFR 23.999	Yes  No  N/A

- Fuel strainer or filter condition					CFR 23.997	Yes  No  N/A

# Propeller Inspection

## Propeller Part number and serial number 				CFR 45.11	Yes  No  N/A

## Part number #_____________________  Serial Number #_________________________

- Is there a propeller maintenance record (log book)		CFR 43.2(a)	Yes  No  N/A

- Propeller seal leaking  					CFR23.907	Yes  No  N/A

- Propeller for nicks, cracks, and damage 			AC 43.13-2B para 8-73 	Yes  No  N/A

- File marks after dressing propeller NOT allowed 		CFR 43.13	Yes  No  N/A

- Repairman or A&P record entry after dressing nicks		CFR 43.9	Yes  No  N/A

- Propeller spinner had doubler added to repair cracks Not ALLOWED CFR 23.907	Yes  No  N/A

- Propeller spinner(s) cracks ___, NO cracks allowed  nuts safety wired __ Missing screws from spinner None allowed ____ Reference Service Letters if cracked:

- a.   McCauley 1992-14C -part must be replaced					Yes  No  N/A

- b.   Hartzell HC-SL-61-91 Requires a Field Approval				Yes  No  N/A

- Sensenich See aircraft maintenance manuals				Yes  No  N/A

- Propeller grinding when rotating 		AC 43.13-1B para 8-2(c )(2)	Yes  No  N/A

- Corrosion pitting on blades or hub None Allowed 		CFR A35.3	Yes  No  N/A

- Paint on propeller blades, type per manufacture manual		CFR 23.609	Yes  No  N/A

- If repainted after rework type of paint applied lacquer base or polyurethane enamel and was it recorded in the propeller maintenance record					CFR 43.5	Yes  No  N/A

- STC propeller check engine gages for new limitations 		CFR A35.4	Yes  No  N/A

- Is the propeller the right diameter / width			CFR 23.45	Yes  No  N/A

- Has the propeller tips been altered (rounded or square)		CFR 43Append A 	Yes  No  N/A

- Are their repairs in the propeller maintenance records		CFR 43.9 	Yes  No  N/A

- Has the shot peen been removed after reworked at hub		CFR 43.9	Yes  No  N/A

- Has the hub seal been replaced (service life)			CFR 43.9	Yes  No  N/A

- Prop Hub is it oil filled and leaking				CFR 35.3	Yes  No  N/A

- When was the last hub overhaul				CFR 35.3	Yes  No  N/A

- Pitting corrosion on Hub NONE ALLOWED			CFR 35.3	Yes  No  N/A

- Hub, blade clamps, and pitch change mechanisms should be inspected for corrosion NONE ALLOWED									CFR A35.3	Yes  No  N/A

- Were new propeller bolts installed				CFR A35.3	Yes  No  N/A

- Were new nuts used on the propeller bolts			CFR A35.3	Yes  No  N/A

- Files marks on blades						CFR A43.5	Yes  No  N/A

- New cotter pins installed in retaining nuts per Manufacture 	CFR A35.4	Yes  No  N/A

- Is the spinner shimmed to the spinner bracket if required 		CFR A35.3	Yes  No  N/A

- Pitch change counterweights on blade clamps should be inspected for security, safety											CFR A35.3	Yes  No  N/A

- Adequate counterweight clearance within the spinner		CFR 23.925	Yes  No  N/A

- Are the propeller blades in track				CFR 23.925	Yes  No  N/A

- De-icer boots for signs of deterioration and security		CFR 23.929	Yes  No  N/A

- Propeller total time is recorded in propeller record 		CFR 91.417(2)	Yes  No  N/A

- Propeller vibration rate					CFR 23.907	Yes  No  N/A

- Propeller clearance to ground and gear				CFR 23.925	Yes  No  N/A

### Special Inspection Areas

- Inverted oil system- remove and inspect check balls, vent line and exhaust valve 	Yes  No  N/A

- Check smoke system attachment, line, injectors and pump			Yes  No  N/A

- Inspect smoke tank for:

- Leaks 									Yes  No  N/A

- Vent line								Yes  No  N/A

- Clean filter								Yes  No  N/A

- Inspect and lube starter and Bendix drive					Yes  No  N/A

- Service top toe brake reservoir							Yes  No  N/A

- Inspect control stick attach bolts						Yes  No  N/A

- No play allowed								Yes  No  N/A

- No wear allowed								Yes  No  N/A

- Cotter pin installed							Yes  No  N/A

- Check stick stops								Yes  No  N/A

- Rod ends No play allowed							Yes  No  N/A

Wings

- Inspect structures’ external and internal conditions				Yes  No  N/A

- Fabric condition/strength							Yes  No  N/A

- Flying/landing wires/attachments/nicks/tension					Yes  No  N/A

- Aileron rigging/hinges/actuators/control stops/trim tabs				Yes  No  N/A

- Interplane/cabane struts/attachments						Yes  No  N/A

- Pitot tube 									Yes  No  N/A

- Inspect aileron hinge play							Yes  No  N/A

- Check Incidence								Yes  No  N/A

- Rod ends and bellcranks- threads, 90 deg and play 				Yes  No  N/A

Empennage

- Inspect structures’ external and internal conditions				Yes  No  N/A

- Fabric condition/strength							Yes  No  N/A

- Brace wires/struts/attachments/tension						Yes  No  N/A

- Inspect/lube hinges and pins- be sure seated properly and no wear			Yes  No  N/A

- Check Control stops								Yes  No  N/A

- Actuator cables and pushrods							Yes  No  N/A

- Trim tab and actuator system							Yes  No  N/A

- Controls/"elevator pushrods reversal bushing welds				Yes  No  N/A

- Remove the rear seat back and carefully inspect for				Yes  No  N/A

- Pivot bushing for the reversal mechanism					Yes  No  N/A

Landing Gear

- Inspect condition/alignment/structure						Yes  No  N/A

- Tire wear and inflation							Yes  No  N/A

- Tailwheel condition and attachment						Yes  No  N/A

- Tailwheel springs, chains, and clips						Yes  No  N/A

- Drain/Bleed brakes								Yes  No  N/A

Operational Check

- Start/full static RPM/idle RPM							Yes  No  N/A

- Mag differential check (1800/175/50) and “hot” mag check			Yes  No  N/A

- Instrument indications- calibrate						Yes  No  N/A

- Engine leak checks								Yes  No  N/A

- Oil consumption history							Yes  No  N/A

- Cycle prop									Yes  No  N/A